Quantum-Aware Continued Training

(QACT) Methodology Guide

Fixing the Environment, Fine-Tuning V4 Seed 3, and Deploying on Qiskit Aer

Supersedes Zero-Shot Transfer Approach

Autonomous RL for Adaptive Resonator Calibration  |  4th Semester Mini Project

Dayananda Sagar College of Engineering, Bengaluru  |  2025-26

# 0. Root-Cause Diagnosis of the Zero-Shot Failures

Before proposing a new methodology, every failure mode in the current implementation must be identified precisely. The five tracking plots and three code files reveal four distinct bugs. These are not limitations of the zero-shot transfer concept — they are implementation errors that would cause any policy, including a retrained one, to fail. These must be fixed before any fine-tuning is attempted.

## 0.1 Bug 1 — Bell Curve Is a Measurement Artefact, Not a Lorentzian

The sweep_test.py script calls _get_quantum_amplitude() in a loop without resetting prev_amp between frequency points. Because the EMA filter uses alpha=0.5, each measurement carries 50% of the prior reading forward. By the time the sweep reaches 520 kHz, prev_amp has accumulated contributions from all 100 prior measurements starting at 480 kHz. The resulting curve — which floors at 0.42 instead of approaching zero — is not a Lorentzian. It is an EMA smear across a frequency axis.

Consequence: Every agent observation of 'amplitude' during training reflected a proper Lorentzian gradient. Every observation during Qiskit deployment reflects a flat, smeared signal. The agent's learned gradient response is meaningless on this input.

## 0.2 Bug 2 — Drift Is Injected Outside the Environment

In deploy_v4_qiskit.py, the line inner.f0_current += np.random.normal(0, 500) executes in the outer loop after env.step(). The environment's step() method does not inject drift. This means the agent receives a reward signal computed at a static qubit frequency, and then the world changes between steps without the agent's observation capturing the change. The agent is calibrating a stationary target, not a drifting one. The visual appearance of 'drift' in the plots comes from the f0_current value changing between steps in the outer loop, but the agent never experienced that change during its step computation.

This single bug invalidates all five tracking plots as measurements of drift-tracking ability. They show static-target tracking with post-hoc drift injection for display purposes.

## 0.3 Bug 3 — Observation Space Formula Mismatch

V4 Seed 3 was trained with amp_gradient = delta_amp / (abs(prev_action) + epsilon), where prev_action is the raw normalised action in [-1, 1]. The current environment computes grad = clip((delta_amp * 5000.0) / (abs(freq_step) + 1.0), -1, 1), where freq_step is the physical frequency shift in Hz. The numerator scaling (5000.0 multiplier) and denominator units (Hz vs normalised) are both wrong. The agent receives a gradient value computed from a completely different formula than what it learned to interpret. The clip to [-1, 1] additionally destroys magnitude information that V4 was specifically trained to exploit.

## 0.4 Bug 4 — Action Slew Limiter Corrupts the Policy

The smoothed_action = 0.1 * raw_action + 0.9 * prev_smoothed_action filter (beta=0.1) means the environment executes approximately 10% of each intended action per step. V4 Seed 3 computes its gradient as delta_amp / abs(prev_action), where prev_action is the smoothed value stored as prev_action_val. But the gradient the policy learned to associate with its raw network output assumed immediate full execution of its actions. The slew limiter decouples the policy's internal model of cause and effect. This is the primary cause of the 'paralysis' behaviour seen in the damped run — the policy cannot build up enough effective frequency shift to follow the drift because 90% of each action is discarded.

| Bug | Severity | Current (Broken) | Fix Required |
| --- | --- | --- | --- |
| EMA in sweep | Reset prev_amp=0 before each measurement in sweep. Separate sweep from tracking. | CRITICAL | prev_amp not reset between sweep points. Floor at 0.42. |
| Drift injection | Drift must be inside step(). step() calls _inject_drift() before returning. | CRITICAL | Injected outside step() in deploy loop. Agent sees static target. |
| Gradient formula | delta_amp / (abs(prev_action_norm) + 1e-6). Identical to V4 training formula. | CRITICAL | clip((delta_amp*5000)/(freq_step+1),-1,1). Wrong units, wrong scale. |
| Action slew limiter | Remove slew limiter entirely. Apply raw clipped action to frequency. | CRITICAL | beta=0.1. Agent executes 10% of intended action per step. |
| omega_rabi value | 5000 Hz. Matches Q=50: half-bandwidth=f0/Q/2=500000/50/2=5000 Hz. | IMPORTANT | 10000 Hz. Curve half-width=10kHz, much wider than Q=50 training. |
| Reward function | Reward = amplitude only. Matches V4 training reward exactly. | IMPORTANT | amp + 0.5*delta_amp*sign(action) - 0.01*abs(action). Modified. |

# 1. Why Fine-Tuning Is Still Needed After Fixing the Environment

Even after correcting all four bugs, a distributional gap will remain between classical simulation and Qiskit Aer. This gap is not a bug — it is a fundamental physics difference. Understanding it precisely is necessary to choose the right fine-tuning strategy.

## 1.1 Shot Noise vs Additive Gaussian Noise

V4 Seed 3 was trained on amplitude observations corrupted by additive Gaussian noise: amp_observed = amp_true + N(0, sigma). Qiskit's shot noise is binomial: with N_shots measurements at true probability p, the observed probability has variance p(1-p)/N_shots. These distributions are different in two important ways. First, Gaussian noise is homoscedastic (constant variance). Binomial shot noise is heteroscedastic — variance is highest near p=0.5 and approaches zero near p=0 and p=1. Near the resonance peak where p is close to 1, shot noise is actually smaller than Gaussian noise. On the flanks where p is around 0.5, shot noise is larger. V4's gradient estimates, which depend on delta_amp, will be systematically biased because the noise profile it expects is wrong.

## 1.2 The Amplitude Floor Problem

With omega_rabi=5000 Hz and the frequency window spanning ±25000 Hz, at the window boundary delta=25000, so P(|1>) = 5000^2 / (5000^2 + 25000^2) = 0.038. The amplitude is effectively near zero at the window edges. V4 was trained with Q=50 on a resonator where the floor also approaches zero. This should match well. However, with shot noise at these low amplitudes, a single measurement at p=0.038 with 1024 shots produces a count of approximately 39 ± 6 — about 16% relative noise. This is much noisier than the training regime at this amplitude level.

## 1.3 Summary: The Residual Gap

After fixing all four bugs, the expected zero-shot performance based on the noise distribution difference is Outcome B from the previous guide — partial transfer with moderate degradation. The near-resonance fraction should be somewhere between 70% and 85%, compared to the simulation's 94.8%. Fine-tuning on the Qiskit environment with a small number of additional timesteps is predicted to close this gap to above 90%.

# 2. Quantum-Aware Continued Training (QACT) — The Correct Methodology

QACT treats the quantum simulator as a new deployment environment that shares the same task structure as the classical training environment but has a different noise distribution. The correct response is not to modify the environment to match the agent (the zero-shot approach's failure mode), but to adapt the agent to the environment through a small number of additional training steps.

QACT consists of four phases executed in strict order:

| Phase | Name | Compute Cost | Output |
| --- | --- | --- | --- |
| Phase 0 | Environment Canonicalisation | None | qiskit_resonator_env_v2.py: clean, correct environment |
| Phase 1 | Baseline Collection | ~30 min | 500 zero-shot episodes. Establishes pre-fine-tuning benchmark. |
| Phase 2 | Quantum Fine-Tuning | 2-4 hours | V4_QFT model: V4 Seed 3 adapted to Qiskit noise |
| Phase 3 | Post-Fine-Tuning Evaluation | ~30 min | 500 evaluation episodes. Comparison table for paper. |

# 3. Phase 0 — Environment Canonicalisation

This phase produces a single file: qiskit_resonator_env_v2.py. It replaces qiskit_resonator_env.py entirely. The new environment has zero engineering hacks. Every parameter that was modified in the original environment is restored to its V4 training equivalent.

## 3.1 Parameter Mapping: Training vs Qiskit

| Parameter | V4 Training Value | Broken Qiskit Value | Corrected Qiskit V2 Value |
| --- | --- | --- | --- |
| Frequency centre | 500000 Hz | 500000 Hz | 500000 Hz (unchanged) |
| Frequency half-range | 25000 Hz | 50000 Hz (multiplier=30k mismatch) | 25000 Hz |
| Action scale | 3000 Hz/unit | 30000 Hz/unit (multiplier) | 3000 Hz/unit |
| Drift sigma | 500 Hz/step inside step() | 500 Hz/step in outer loop (broken) | 500 Hz/step inside step() |
| EMA alpha | 0.3 | 0.5 (over-smoothed) | 0.3 |
| Gradient formula | delta_amp/(¦prev_action¦+1e-6) | clip(delta_amp*5000/(freq_step+1),-1,1) | delta_amp/(¦prev_action¦+1e-6) |
| Action slew | None (raw action) | beta=0.1 (90% discarded) | None — raw action applies fully |
| Reward | amplitude only | amp+0.5*delta*sign(a)-0.01*¦a¦ | amplitude only |
| omega_rabi | N/A (Q=50 Lorentzian) | 10000 Hz (too wide) | 5000 Hz (matches Q=50 half-bandwidth) |
| Shots | N/A (noiseless) | 4096 | 1024 for training, 2048 for eval |

## 3.2 omega_rabi Calibration — Why 5000 Hz

The V4 training Lorentzian has Q=50 and f0=500000 Hz. The full 3dB bandwidth is f0/Q = 10000 Hz. This means the half-bandwidth (the frequency offset at which amplitude drops to 0.5) is 5000 Hz. In the Rabi physics model, P(|1>) = omega^2 / (omega^2 + delta^2) drops to 0.5 when delta = omega_rabi. Therefore omega_rabi = 5000 Hz exactly matches the Q=50 half-bandwidth. This ensures the Qiskit Lorentzian has the same sharpness and the same gradient profile that V4 learned to navigate.

## 3.3 Complete Canonical Environment Code

The following is the complete qiskit_resonator_env_v2.py. Every line is either identical to rl_environment.py (V4) or is the direct Qiskit equivalent.

# qiskit_resonator_env_v2.py

# CANONICAL version — no hacks, no engineering multipliers

# All parameters match V4 training exactly

import gymnasium as gym

import numpy as np

from qiskit import QuantumCircuit

from qiskit_aer import AerSimulator

class QiskitResonatorEnvV2(gym.Env):

def __init__(

self,

f0_center   = 500000.0,  # Hz — matches training

f_range     =  25000.0,  # Hz half-range — matches training

drift_sigma =    500.0,  # Hz/step — matches training

action_scale=   3000.0,  # Hz/unit — matches training

ema_alpha   =      0.3,  # — matches training

omega_rabi  =   5000.0,  # Hz — calibrated to Q=50

shots       =   1024,    # quantum measurements per step

seed        =     42

):

super().__init__()

self.f0_center    = f0_center

self.f_range      = f_range

self.drift_sigma  = drift_sigma

self.action_scale = action_scale

self.ema_alpha    = ema_alpha

self.omega_rabi   = omega_rabi

self.shots        = shots

self.rng          = np.random.default_rng(seed)

self.simulator    = AerSimulator()

# Internal state

self.f0_current   = f0_center   # hidden qubit transition freq

self.f_probe      = f0_center   # agent's current probe freq

self.prev_amp     = 0.0

self.prev_action  = 0.0         # normalised, in [-1,1]

self.ema_amp      = 0.0

self.delta_amp    = 0.0

self.step_count   = 0

self.max_steps    = 200

# Observation and action spaces — IDENTICAL to V4

obs_lo = np.array([-1,-1,-1,-2,-10], dtype=np.float32)

obs_hi = np.array([ 2, 1, 1, 2, 10], dtype=np.float32)

self.observation_space = gym.spaces.Box(obs_lo, obs_hi, dtype=np.float32)

self.action_space = gym.spaces.Box(

low=-1.0, high=1.0, shape=(1,), dtype=np.float32)

def _quantum_amplitude(self, f_probe_hz):

"""

Rabi physics mapping — correct formula from Section 2 of

the integration guide. Returns EMA-filtered P(|1>).

"""

delta = f_probe_hz - self.f0_current

theta = float(np.pi * self.omega_rabi /

np.sqrt(self.omega_rabi**2 + delta**2))

qc = QuantumCircuit(1, 1)

qc.rx(theta, 0)

qc.measure(0, 0)

result = self.simulator.run(qc, shots=self.shots).result()

counts = result.get_counts()

raw_p1 = counts.get('1', 0) / self.shots

# EMA filter — alpha=0.3 matches V4 training

self.ema_amp = (self.ema_alpha * raw_p1 +

(1 - self.ema_alpha) * self.ema_amp)

return float(self.ema_amp)

def _inject_drift(self):

"""

Gaussian drift on f0_current. Called INSIDE step().

This is the fix for Bug 2 — drift must be internal.

"""

shift = float(self.rng.normal(0.0, self.drift_sigma))

self.f0_current = float(np.clip(

self.f0_current + shift,

self.f0_center - self.f_range,

self.f0_center + self.f_range

))

def _build_obs(self, amplitude):

"""

5-element observation — IDENTICAL formula to V4 rl_environment.py.

Do not modify any element without also modifying the training env.

"""

amp = float(amplitude)

# Element 1: frequency error normalised to [-1, 1]

freq_err_norm = float(

(self.f_probe - self.f0_center) / self.f_range

)

# Element 2: previous normalised action

prev_a = float(self.prev_action)

# Element 3: delta_amp

self.delta_amp = float(amp - self.prev_amp)

# Element 4: amp_gradient — V4's key contribution

# IDENTICAL formula to training. Do not rescale.

eps = 1e-6

amp_grad = float(np.clip(

self.delta_amp / (abs(self.prev_action) + eps),

-10.0, 10.0

))

return np.array(

[amp, freq_err_norm, prev_a, self.delta_amp, amp_grad],

dtype=np.float32

)

def reset(self, seed=None, options=None):

super().reset(seed=seed)

# Start qubit frequency near centre with small random offset

self.f0_current = float(self.rng.normal(

self.f0_center, self.f_range * 0.1

))

self.f0_current = float(np.clip(

self.f0_current,

self.f0_center - self.f_range,

self.f0_center + self.f_range

))

self.f_probe      = float(self.f0_center)

self.prev_amp     = 0.0

self.prev_action  = 0.0

self.ema_amp      = 0.0

self.delta_amp    = 0.0

self.step_count   = 0

amp = self._quantum_amplitude(self.f_probe)

self.prev_amp = amp

obs = self._build_obs(amp)

return obs, {}

def step(self, action):

# 1. Apply raw action — NO slew limiter

action_val = float(np.clip(action[0], -1.0, 1.0))

# 2. Update probe frequency

delta_freq = action_val * self.action_scale

self.f_probe = float(np.clip(

self.f_probe + delta_freq,

self.f0_center - self.f_range,

self.f0_center + self.f_range

))

# 3. Inject drift (INSIDE step — Bug 2 fix)

self._inject_drift()

# 4. Measure qubit excitation

amp = self._quantum_amplitude(self.f_probe)

# 5. Build observation

obs = self._build_obs(amp)

# 6. Reward — amplitude only, identical to V4

reward = float(amp)

# 7. Update state

self.prev_amp    = amp

self.prev_action = action_val

self.step_count += 1

terminated = self.step_count >= self.max_steps

truncated  = False

info = {

'f_probe':       self.f_probe,

'f_qubit':       self.f0_current,

'freq_error_hz': abs(self.f_probe - self.f0_current),

'amplitude':     amp

}

return obs, reward, terminated, truncated, info

## 3.4 Sweep Verification Test

Before Phase 1, run this corrected sweep test. The plot must show a proper Lorentzian with peak at 500 kHz and floor near 0.038 at the window edges. If it does, Phase 0 is complete. If not, stop and debug the omega_rabi value.

# sweep_verify_v2.py

import numpy as np, matplotlib.pyplot as plt

from qiskit_integration.qiskit_resonator_env_v2 import QiskitResonatorEnvV2

env = QiskitResonatorEnvV2(shots=2048)

env.f0_current = 500000.0

freqs = np.linspace(475000, 525000, 80)

amps  = []

for f in freqs:

env.ema_amp = 0.0            # RESET between each point

env.prev_amp = 0.0           # RESET between each point

# Single raw measurement, no EMA carry-forward

delta = f - env.f0_current

theta = float(np.pi * env.omega_rabi /

np.sqrt(env.omega_rabi**2 + delta**2))

from qiskit import QuantumCircuit

qc = QuantumCircuit(1,1)

qc.rx(theta, 0)

qc.measure(0, 0)

r = env.simulator.run(qc, shots=2048).result()

amps.append(r.get_counts().get('1',0)/2048)

plt.figure(figsize=(9,4))

plt.plot(freqs/1000, amps, 'b-', lw=2)

plt.axvline(500, color='r', ls='--', label='f0 = 500 kHz')

plt.axhline(0.5, color='grey', ls=':', label='P=0.5 (half-max)')

plt.xlabel('Frequency (kHz)')

plt.ylabel('P(|1>)')

plt.title('Qiskit V2 Lorentzian — should peak at 500 kHz, floor ~0.04')

plt.legend(); plt.grid(True, alpha=0.3)

plt.savefig('sweep_v2_verification.png', dpi=150)

Expected output: Peak exactly at 500 kHz reaching P=0.98-1.0 (shot noise limited). At 475 kHz and 525 kHz, P should be approximately 0.038. The half-maximum points (P=0.5) should be at approximately 495 kHz and 505 kHz, confirming the 10 kHz bandwidth matching Q=50.

# 4. Phase 1 — Zero-Shot Baseline Collection

This phase runs V4 Seed 3 on the corrected canonical environment WITHOUT any fine-tuning. This establishes the true zero-shot performance baseline — the number the paper will compare against the fine-tuned result. Do not skip this phase. If you fine-tune without a baseline, you have no before/after comparison for the paper.

## 4.1 Baseline Script

# phase1_baseline.py

import os, json, csv, numpy as np

from stable_baselines3 import PPO

from stable_baselines3.common.vec_env import DummyVecEnv, VecNormalize

from qiskit_integration.qiskit_resonator_env_v2 import QiskitResonatorEnvV2

MODEL_PATH  = r'C:\MiniProject_Sem4\rl_training\trained_models'

r'\v4_gradient_obs\seed_3\best_model'

STATS_PATH  = r'C:\MiniProject_Sem4\rl_training\trained_models'

r'\v4_gradient_obs\seed_3\vec_normalize.pkl'

OUT_DIR     = r'C:\MiniProject_Sem4\qiskit_integration\results'

os.makedirs(OUT_DIR, exist_ok=True)

N_EPISODES = 500

def make_env():

return DummyVecEnv([lambda: QiskitResonatorEnvV2(shots=1024)])

env = make_env()

env = VecNormalize.load(STATS_PATH, env)

env.training = False

env.norm_reward = False

model = PPO.load(MODEL_PATH, env=env)

print(f'Running {N_EPISODES} zero-shot baseline episodes...')

results = []

for ep in range(N_EPISODES):

obs = env.reset()

ep_amp, ep_err = [], []

done = False

while not done:

action, _ = model.predict(obs, deterministic=True)

obs, _, done, info = env.step(action)

ep_amp.append(float(info[0]['amplitude']))

ep_err.append(float(info[0]['freq_error_hz']))

results.append({

'mae':   float(np.mean(ep_err)),

'near':  float(np.mean(np.array(ep_amp) > 0.90)),

'low':   float(np.mean(np.array(ep_amp) < 0.70))

})

if (ep+1) % 100 == 0:

print(f'  {ep+1}/{N_EPISODES} | MAE {np.mean([r["mae"] for r in results]):.0f} Hz')

summary = {

'phase': 'zero_shot_baseline',

'model': 'V4_Seed3_original',

'mae_mean':  float(np.mean([r['mae']  for r in results])),

'mae_std':   float(np.std( [r['mae']  for r in results])),

'near_mean': float(np.mean([r['near'] for r in results])),

'low_mean':  float(np.mean([r['low']  for r in results]))

}

with open(os.path.join(OUT_DIR,'baseline_summary.json'),'w') as f:

json.dump(summary, f, indent=2)

print('Baseline complete:', summary)

This phase takes approximately 30 minutes on an RTX 3050. N_EPISODES=500 gives a reliable statistical baseline. Do not reduce below 200 episodes.

# 5. Phase 2 — Quantum-Aware Fine-Tuning (QAFT)

This phase loads V4 Seed 3 and continues PPO training on the canonical Qiskit environment. The learning rate is reduced by 5x from the original training value. This prevents the fine-tuning from destroying the simulation-learned policy while allowing it to adapt to quantum shot noise.

## 5.1 Fine-Tuning Rationale

The original V4 training used learning_rate=2.5e-4. The fine-tuning uses learning_rate=5e-5. This follows directly from the hardware integration plan's Option A strategy, adapted for Qiskit instead of physical hardware. At 5e-5, the policy makes small corrective adjustments to its existing behaviour rather than relearning from scratch. The policy has already learned the core task structure (gradient-following, directional tracking). What it needs to learn is the shot-noise statistics, specifically that the amplitude signal is noisier at mid-range P values and should be weighted accordingly.

Training budget: 100,000 to 200,000 timesteps. With 200-step episodes, this is 500 to 1000 additional episodes. At approximately 3-4 seconds per episode on an RTX 3050 (Qiskit simulation is CPU-bound), total training time is 25-70 minutes. This is a one-time cost.

## 5.2 VecNormalize Strategy

Two options exist for handling VecNormalize during fine-tuning:

- Option A — Frozen normalisation: Load vec_normalize.pkl with training=False. The running statistics are not updated. The policy receives observations normalised by simulation-era statistics. This is safest and is recommended for budgets under 150k timesteps.

- Option B — Live normalisation: Load vec_normalize.pkl with training=True. The statistics slowly update toward Qiskit-era values. This gives better long-run adaptation but risks destabilising the policy if the Qiskit distribution differs too much in early episodes. Use only if frozen normalisation shows insufficient improvement after 150k steps.

Start with Option A (frozen). Only switch to Option B if MAE after 150k steps has not improved by at least 20% over the zero-shot baseline.

## 5.3 Fine-Tuning Script

# phase2_finetune.py

import os, numpy as np

from stable_baselines3 import PPO

from stable_baselines3.common.vec_env import DummyVecEnv, VecNormalize

from stable_baselines3.common.callbacks import EvalCallback

from qiskit_integration.qiskit_resonator_env_v2 import QiskitResonatorEnvV2

MODEL_PATH  = r'C:\MiniProject_Sem4\rl_training\trained_models'

r'\v4_gradient_obs\seed_3\best_model'

STATS_PATH  = r'C:\MiniProject_Sem4\rl_training\trained_models'

r'\v4_gradient_obs\seed_3\vec_normalize.pkl'

SAVE_DIR    = r'C:\MiniProject_Sem4\qiskit_integration\v4_qft'

os.makedirs(SAVE_DIR, exist_ok=True)

TOTAL_TIMESTEPS = 150000   # Start here. Increase to 200k if needed.

LR_FINETUNE     = 5e-5     # 5x lower than original 2.5e-4

N_ENVS          = 1        # Single env — Qiskit is CPU-bound

# Training environment

def make_train_env():

return DummyVecEnv([lambda: QiskitResonatorEnvV2(

shots=512,   # Fewer shots during training = faster iterations

seed=0       # Fixed seed for reproducibility

)])

# Evaluation environment (more shots = cleaner signal for callback)

def make_eval_env():

return DummyVecEnv([lambda: QiskitResonatorEnvV2(

shots=1024,

seed=99

)])

# Load training environment and normalisation

train_env = make_train_env()

train_env = VecNormalize.load(STATS_PATH, train_env)

train_env.training = False  # Option A: frozen normalisation

train_env.norm_reward = False

# Load evaluation environment (no normalisation needed for callback)

eval_env = make_eval_env()

# Load V4 Seed 3 and update learning rate

model = PPO.load(MODEL_PATH, env=train_env)

model.learning_rate = LR_FINETUNE

# Reset the optimizer state so old gradients don't interfere

model.policy.optimizer.param_groups[0]['lr'] = LR_FINETUNE

print(f'Fine-tuning V4 Seed 3 on Qiskit Aer')

print(f'LR: {LR_FINETUNE} | Steps: {TOTAL_TIMESTEPS} | Shots: 512/step')

# EvalCallback saves best model during fine-tuning

eval_cb = EvalCallback(

eval_env,

best_model_save_path=SAVE_DIR,

log_path=os.path.join(SAVE_DIR, 'logs'),

eval_freq=5000,           # Evaluate every 5000 timesteps

n_eval_episodes=20,       # Quick eval during training

deterministic=True,

render=False

)

# Fine-tune

model.learn(

total_timesteps=TOTAL_TIMESTEPS,

callback=eval_cb,

reset_num_timesteps=False  # Preserve step count from original training

)

# Save final model and normalisation stats

model.save(os.path.join(SAVE_DIR, 'v4_qft_final'))

train_env.save(os.path.join(SAVE_DIR, 'vec_normalize_qft.pkl'))

print('Fine-tuning complete. Model saved to:', SAVE_DIR)

## 5.4 Shot Budget During Training vs Evaluation

Using shots=512 during training deliberately introduces more shot noise per observation than the evaluation setting of shots=1024. This is a form of data augmentation — training under noisier conditions forces the policy to become more robust. The evaluation then runs at 1024 shots for fairer comparison with the zero-shot baseline. Do not use shots=128 or fewer; below 200 shots the Rabi curve distortion from shot noise becomes too severe for the gradient signal to be reliable.

# 6. Phase 3 — Post-Fine-Tuning Evaluation and Comparison

This phase runs the fine-tuned V4_QFT model on the same canonical environment for 500 episodes and compares against both the zero-shot baseline (Phase 1) and the original simulation results.

## 6.1 Evaluation Script

# phase3_evaluate.py

import os, json, numpy as np

from stable_baselines3 import PPO

from stable_baselines3.common.vec_env import DummyVecEnv, VecNormalize

from qiskit_integration.qiskit_resonator_env_v2 import QiskitResonatorEnvV2

QFT_MODEL_PATH = r'C:\MiniProject_Sem4\qiskit_integration\v4_qft\best_model'

QFT_STATS_PATH = r'C:\MiniProject_Sem4\qiskit_integration\v4_qft\vec_normalize_qft.pkl'

OUT_DIR        = r'C:\MiniProject_Sem4\qiskit_integration\results'

N_EPISODES = 500

env = DummyVecEnv([lambda: QiskitResonatorEnvV2(shots=1024)])

env = VecNormalize.load(QFT_STATS_PATH, env)

env.training = False; env.norm_reward = False

model = PPO.load(QFT_MODEL_PATH, env=env)

print(f'Evaluating V4_QFT over {N_EPISODES} episodes...')

results, trajectories = [], []

for ep in range(N_EPISODES):

obs = env.reset()

ep_amp, ep_err = [], []

ep_f_probe, ep_f_qubit = [], []

done = False

while not done:

action, _ = model.predict(obs, deterministic=True)

obs, _, done, info = env.step(action)

ep_amp.append(float(info[0]['amplitude']))

ep_err.append(float(info[0]['freq_error_hz']))

ep_f_probe.append(float(info[0]['f_probe']))

ep_f_qubit.append(float(info[0]['f_qubit']))

results.append({

'mae':  float(np.mean(ep_err)),

'near': float(np.mean(np.array(ep_amp) > 0.90)),

'low':  float(np.mean(np.array(ep_amp) < 0.70))

})

if ep < 5:   # Save first 5 trajectories for plotting

trajectories.append({

'f_probe': ep_f_probe,

'f_qubit': ep_f_qubit,

'amp': ep_amp

})

summary = {

'phase': 'post_finetune_eval',

'model': 'V4_QFT',

'mae_mean':  float(np.mean([r['mae']  for r in results])),

'mae_std':   float(np.std( [r['mae']  for r in results])),

'near_mean': float(np.mean([r['near'] for r in results])),

'near_std':  float(np.std( [r['near'] for r in results])),

'low_mean':  float(np.mean([r['low']  for r in results]))

}

with open(os.path.join(OUT_DIR,'qft_summary.json'),'w') as f:

json.dump(summary, f, indent=2)

print('QFT evaluation complete:', summary)

## 6.2 Three-Way Comparison Table Template

Fill in the Qiskit columns after running Phases 1 and 3. The simulation column is fixed from existing results.

| Model | Environment | MAE (Hz) | Amp >0.90 | Amp <0.70 | Notes |
| --- | --- | --- | --- | --- | --- |
| V4 Seed 3 | Classical sim (Gaussian) | 1510 ± 367 | ~94.8% | ~0.4% | Locked. Original training result. |
| V4 Seed 3 | Qiskit Aer (zero-shot) | Run Phase 1 | Run Phase 1 | Run Phase 1 | Baseline before fine-tuning. |
| V4_QFT | Qiskit Aer (fine-tuned) | Run Phase 3 | Run Phase 3 | Run Phase 3 | Expected: MAE < 2x simulation. |

## 6.3 Trajectory Visualisation Script

# plot_trajectories.py

import json, numpy as np, matplotlib.pyplot as plt

from matplotlib.gridspec import GridSpec

# Load a trajectory saved from phase3_evaluate.py

# (Add save_trajectories=True flag and load from json)

fig = plt.figure(figsize=(12,8))

gs = GridSpec(2, 1, figure=fig, hspace=0.35)

ax1 = fig.add_subplot(gs[0])

ax1.plot(np.array(traj['f_qubit'])/1000,'r--',lw=1.5,alpha=0.8,label='Qubit freq (hidden drift)')

ax1.plot(np.array(traj['f_probe'])/1000,'b-', lw=2,   label='Agent probe frequency')

ax1.set_ylabel('Frequency (kHz)'); ax1.legend(); ax1.grid(True,alpha=0.3)

ax1.set_title('V4_QFT: Quantum-Aware Fine-Tuned Tracking')

ax2 = fig.add_subplot(gs[1])

ax2.plot(traj['amp'],'g-',lw=1.5,label='P(|1>)')

ax2.axhline(0.9,color='k',ls=':',label='0.90 threshold')

ax2.set_ylabel('Excitation Prob.'); ax2.set_xlabel('Time Steps')

ax2.set_ylim(0,1.05); ax2.legend(); ax2.grid(True,alpha=0.3)

plt.savefig('v4_qft_trajectory.png', dpi=200, bbox_inches='tight')

# 7. Gemini Implementation Prompt

Copy the following prompt verbatim to Gemini for code implementation. Do not ask Gemini to design the approach — the approach is fully specified above. Only ask Gemini to implement it exactly.

GEMINI PROMPT — BEGIN

You are a Python code implementation assistant working on a reinforcement

learning project. I will give you exact specifications. Implement them

precisely. Do not add features, change parameter values, or refactor

working code. Only write what is explicitly requested.

PROJECT CONTEXT:

- Project: Autonomous RL for Adaptive Resonator Calibration

- Trained model: V4 Seed 3, PPO, stable-baselines3

- Model path: C:\MiniProject_Sem4\rl_training\trained_models\v4_gradient_obs\seed_3\

- Conda env: quantum_control (Python 3.10, torch 2.1.2+cu118, qiskit, qiskit-aer)

- Working directory: C:\MiniProject_Sem4\qiskit_integration\

TASK 1 — Create qiskit_resonator_env_v2.py

Create a new file at the path above with the EXACT code below.

Do not modify any parameter value. Do not add any smoothing, slew limiting,

action damping, gradient rescaling, or reward shaping.

[PASTE THE COMPLETE QiskitResonatorEnvV2 CLASS FROM SECTION 3.3 HERE]

TASK 2 — Create sweep_verify_v2.py

Create a new file with the sweep verification code from Section 3.4.

Do not add EMA carry-forward. Each frequency point must be measured

with a freshly initialised circuit. Run this script and tell me:

a) What frequency the peak occurs at (should be 500 kHz)

b) What P value the peak reaches (should be 0.95 to 1.0)

c) What P value the edges at 475 and 525 kHz show (should be ~0.04)

If any of these conditions fail, stop and report the failure.

TASK 3 — Create phase1_baseline.py

Create the baseline evaluation script from Section 4.1 exactly.

Run 500 episodes with shots=1024 on the canonical V2 environment.

Use deterministic=True for inference.

Save results to C:\MiniProject_Sem4\qiskit_integration\results\baseline_summary.json

Report the four numbers: mae_mean, mae_std, near_mean, low_mean.

TASK 4 — Create phase2_finetune.py

Create the fine-tuning script from Section 5.3 exactly.

Use learning_rate=5e-5 (do NOT change this value).

Use shots=512 for training, shots=1024 for eval callback.

Use total_timesteps=150000.

Use reset_num_timesteps=False.

Set train_env.training=False (frozen normalisation).

Save the fine-tuned model to:

C:\MiniProject_Sem4\qiskit_integration\v4_qft\

Run fine-tuning and report the final EvalCallback mean_reward.

TASK 5 — Create phase3_evaluate.py

Create the post-fine-tuning evaluation script from Section 6.1 exactly.

Run 500 episodes with shots=1024. Use deterministic=True.

Save results to: C:\MiniProject_Sem4\qiskit_integration\results\qft_summary.json

Report all six numbers from the summary dict.

IMPORTANT CONSTRAINTS FOR ALL TASKS:

1. Do not modify any parameter value specified above.

2. Do not add slew limiting, action smoothing, or gradient rescaling.

3. Do not change the reward function from: reward = float(amplitude).

4. Do not retrain from scratch — always load from the seed_3 checkpoint.

5. If any task fails, report the exact error and stop. Do not attempt workarounds.

6. Complete each task in order. Do not begin Task N+1 until Task N succeeds.

GEMINI PROMPT — END

# 8. Expected Outcomes and Paper Narrative

Based on the corrected environment and fine-tuning methodology, the following outcome ranges are predicted. The paper narrative for each case is included.

## 8.1 Zero-Shot Baseline (Phase 1) — Expected Range

| Metric | Expected Range | Basis |
| --- | --- | --- |
| MAE (Hz) | 1800 to 3000 Hz | Binomial shot noise vs Gaussian training noise. 1-2x degradation from simulation. |
| Amp >0.90 | 70% to 85% | Shot noise at mid-range P reduces gradient reliability. Partial transfer. |
| Amp <0.70 | 1% to 5% | V4's gradient obs prevents catastrophic failure. Rare low-amplitude episodes. |
| MAE std | 400 to 800 Hz | Higher variance than simulation due to noise distribution mismatch. |

## 8.2 Post-Fine-Tuning (Phase 3) — Expected Range

| Metric | Expected Range | Success Criterion |
| --- | --- | --- |
| MAE (Hz) | 1200 to 2000 Hz | Within 2x of simulation MAE (1510 Hz). Below 3000 Hz is acceptable. |
| Amp >0.90 | 85% to 95% | Matching or approaching simulation's 94.8% is a strong result. |
| Amp <0.70 | < 2% | Should not regress from zero-shot baseline. |
| Improvement over zero-shot | >30% MAE reduction | This is the paper's primary claim for the QACT contribution. |

## 8.3 Paper Narrative

The QACT experiment adds two contributions to the paper. The first contribution is the zero-shot baseline result, which establishes the structural alignment between the classical analogue and quantum circuit simulation. Even without adaptation, V4 Seed 3 maintains effective tracking because the Rabi physics Lorentzian is isomorphic to the training Lorentzian and the gradient observation encodes the correct directional information.

The second contribution is the fine-tuning result, which demonstrates that the distributional gap between additive Gaussian noise and quantum shot noise can be closed with a small number of additional training steps at reduced learning rate. This is presented as the sim-to-quantum analogue of the hardware sim-to-real fine-tuning described in the hardware integration plan. The structural parallel between sim-to-hardware and sim-to-quantum adaptation is itself a novel contribution — it suggests the same fine-tuning methodology works across both deployment targets.

The QACT methodology closes the paper's scientific loop: V4 is trained on a classical mathematical model, validated on a quantum physics simulator, and deployed on classical hardware. All three domains share the same observation space structure and the same fine-tuning protocol. This is the architectural generality claim.

# 9. Execution Checklist

- Confirm qiskit and qiskit-aer are installed in quantum_control environment.

- Create qiskit_resonator_env_v2.py — no modifications from Section 3.3.

- Run sweep_verify_v2.py. Confirm peak at 500 kHz, floor ~0.04, half-max at +-5 kHz.

- If sweep fails, debug omega_rabi before proceeding. Do not move forward.

- Run phase1_baseline.py — 500 episodes, shots=1024. Record all four metrics.

- Inspect the baseline. If near_mean > 0.90, zero-shot is already successful. Fine-tuning is still worthwhile but the paper claim changes from 'adaptation required' to 'adaptation improves'.

- Run phase2_finetune.py — 150k timesteps, LR=5e-5, shots=512 train / 1024 eval.

- Monitor EvalCallback mean_reward. It should improve within the first 20k-30k steps. If it plateaus immediately, check that drift is inside step() and not the outer loop.

- After fine-tuning, run phase3_evaluate.py — 500 episodes, shots=1024.

- Fill in the three-way comparison table from Section 6.2.

- Run plot_trajectories.py on the first 5 saved episodes. Confirm blue line tracks red line.

- Record all metrics. Write the paper section using the narrative from Section 8.3.

Do not attempt any of the three remaining simulation tasks (V4a/V4b ablation, robustness test at 750/1000 Hz, sample efficiency curves) concurrently with this Qiskit work. Complete QACT fully first, then return to those tasks.