PROJECT STATUS REPORT

Autonomous Reinforcement Learning for Resonator Calibration:

A Quantum-Inspired Control System (QRC / ARCS)

Resumption Checkpoint — Prepared after a 2-Month Pause

Date: 13 July 2026

| Course Code | 22ET49 ¦ AY 2025–26 |
| --- | --- |
| Institution | Dayananda Sagar College of Engineering (DSCE), Bangalore — VTU Affiliation |
| Department | Electronics and Telecommunication Engineering — 4th Semester |
| Student | Akash Kishore (1DS24ET012) |
| Team | Aadidev Raja K, Adithi H P, Akash Kishore, Khushi Patil |
| Guide | Dr. Jugul Kishore |
| Target Venue | IEEE Access (Q2) — publication-quality target |

# 1. Executive Summary

This report re-establishes full project context after a two-month pause. It separates completed, locked deliverables from open work, and adds a dedicated software fine-tuning section — recommendations only, none of which touch the locked V4 result.

### 1.1 Where the project stands

- RL simulation (core deliverable): COMPLETE and LOCKED. V4 Seed 3 is the authoritative model.

- QACT (Qiskit Aer Quantum Agent Control Test): FORMALLY STOPPED with a documented sim-to-real result.

- Analog simulation (LTSpice): COMPLETE for Stages 1, 3, and 4 (one caveat noted on Stage 4).

- Digital simulation (Wokwi, replacing Proteus 8.5): PENDING — Stages 2 (DAC) and 5 (ADC) not yet built.

- Physical hardware: NOT STARTED. Two procurement blockers remain outstanding.

- Academic paper: NOT STARTED — drafting depends on hardware/HIL results for the full sim-to-real narrative.

### 1.2 Reading this document

Section 2 is a scoreboard of every workstream. Section 3 details what is fully finished and why it is locked. Section 4 details everything still open, in priority order. Section 5 — the section you specifically asked for — lays out software fine-tuning options for the RL/QACT stack, framed as future-work improvements for the paper, not retroactive changes to V4 Seed 3. Section 6 covers hardware and procurement. Section 7 is the phase-by-phase integration roadmap. Section 8 recaps the key technical lessons learned so far. Section 9 is an immediate-action checklist to restart the project this week.

# 2. Status Scoreboard

| Workstream | Status | Key Result / Blocker |
| --- | --- | --- |
| RL Simulation (PPO) | COMPLETE | V4 Seed 3: MAE 1510 ± 367 Hz, 94.8% near-resonance (Gaussian drift) |
| QACT (Qiskit Aer) | STOPPED | v5 final: MAE 3237 ± 4581 Hz, 34.3% near-resonance (~55% rel. improvement over 22.1% baseline) |
| LTSpice — Stage 1 (Power) | COMPLETE | AMS1117 rail confirmed at 3.30073 V |
| LTSpice — Stage 3 (Resonator) | COMPLETE | f0 = 555.61 kHz, Lorentzian peak confirmed |
| LTSpice — Stage 4 (Envelope Detector) | COMPLETE* | *Op-amp supply wiring caveat noted — verify before hardware build |
| Wokwi — Stage 2 (DAC, MCP4921) | PENDING | Not a native Wokwi part — raw SPI.h + transfer16() approach identified |
| Wokwi — Stage 5 (ADC, MCP3202) | PENDING | Not a native Wokwi part — SPI approach still to be worked out |
| Hardware Procurement | BLOCKED | BB909/MV209 varactor not ordered; 1µF C0G caps for PWM filter out of stock |
| Breadboard Build | NOT STARTED | Depends on procurement + Wokwi validation |
| IEEE Access Paper | NOT STARTED | Waiting on HIL / hardware data for sim-to-real section |

# 3. Completed and Locked Work

Everything in this section is under a strict do-not-retrain / do-not-re-simulate policy. Results are considered final for reporting and publication purposes.

## 3.1 RL Simulation — V4 Seed 3 (Authoritative Model)

- Task: PPO agent tracks the resonant frequency of a classical LC resonator (~556 kHz) using amplitude feedback only — no direct frequency measurement, mirroring superconducting-qubit calibration constraints.

- Result: MAE 1510 ± 367 Hz, 94.8% near-resonance fraction under Gaussian drift.

- V4 statistically outperforms V3 and the PID baseline under Mann–Whitney U testing for the Gaussian drift regime (p < 0.001).

- Under the Ornstein–Uhlenbeck (OU) drift regime, the V4 vs. PID difference was not statistically significant — flagged for the fine-tuning discussion in Section 5.

- Verification method: numeric MAE was cross-checked against visual trajectory inspection (agent path vs. reference) to rule out degenerate static policies that can otherwise inflate apparent accuracy.

## 3.2 QACT — Qiskit Aer Quantum Agent Control Test (Stopped)

- Final released version: QACT v5.

- Result: MAE 3237 ± 4581 Hz, 34.3% near-resonance — versus a 22.1% zero-shot baseline, a ~55% relative improvement.

- Framing for the paper: this is documented as a sim-to-real transfer data point (classical-sim-trained policy applied to a quantum-simulator environment), not as a failed experiment. The large standard deviation (4581 Hz) is itself evidence worth discussing — see Section 5.4.

- Status: formally closed — no further QACT runs are planned inside the current experiment set.

## 3.3 LTSpice Analog Simulations

- Stage 1 — Power Supply (AMS1117-3.3): confirmed output 3.30073 V, within spec.

- Stage 3 — LC Resonator: measured resonance f0 = 555.61 kHz with a confirmed Lorentzian response peak, matching the ~556 kHz design target.

- Stage 4 — Envelope Detector: simulation complete, but carries an open caveat on op-amp supply wiring that must be re-checked before this stage is trusted on the physical board (see Section 6.3).

# 4. Pending Work (Priority Order)

## 4.1 Wokwi Digital Simulations — Stages 2 & 5 (Highest Priority)

Proteus 8.5 has been replaced by Wokwi for all digital/microcontroller-side simulation. This is the immediate next task and is the main thing blocking any hardware work.

### Stage 2 — DAC (MCP4921, 12-bit SPI)

- MCP4921 is not a native Wokwi component — it must be driven manually.

- Confirmed approach: use raw Arduino SPI.h with transfer16() rather than the MCP_DAC library, which caused build failures.

- Remaining work: build and verify the Wokwi circuit, confirm output waveform against the LTSpice Stage 1/3 expectations, and lock the pin mapping (ESP32 VSPI — SCK=GPIO18, MOSI=GPIO23, CS=GPIO5).

### Stage 5 — ADC (MCP3202, 12-bit SPI)

- MCP3202 is also not a native Wokwi part; the same raw-SPI strategy should be validated for the read path.

- Remaining work: implement and verify SPI read timing on ESP32 HSPI (SCK=GPIO14, MISO=GPIO12, MOSI=GPIO13, CS=GPIO15), and confirm 12-bit resolution is preserved end-to-end.

### Suggested acceptance criteria before declaring Stages 2 & 5 done

- DAC output settles to the commanded voltage within one PWM/SPI update cycle in simulation.

- ADC read-back of a known injected signal matches expected code within ±1 LSB in simulation.

- Both stages run concurrently in a single Wokwi sketch without SPI bus contention (VSPI vs. HSPI separation verified).

## 4.2 Hardware Procurement

- BB909 or MV209 varactor diode: not yet ordered. BB135,115 (NXP) is the confirmed locally available substitute if lead time on BB909/MV209 is an issue — do not substitute BB910, which was already identified as the wrong part.

- 1 µF C0G capacitors for the PWM RC filter: previously out of stock at the primary source (robu.in) — recheck stock or source an alternative C0G-dielectric supplier. X7R is not an acceptable substitute for this or the varactor coupling capacitors.

- Recommended action this week: place/confirm both orders in parallel with the Wokwi work so hardware build isn't gated on shipping time later.

## 4.3 Physical Breadboard Build (Not Started)

- Cannot begin until Stages 2 and 5 are validated in Wokwi and both procurement items arrive.

- Constraint to respect from day one: 3 cm maximum lead length in the RF path — daisy-chained 400-point breadboards violate this and should not be used for the resonator/RF section.

- Re-verify the Stage 4 op-amp supply wiring caveat physically before trusting envelope-detector output on hardware.

## 4.4 Academic Paper (Not Started)

- Drafting is intentionally sequenced after HIL/hardware data becomes available, since the paper's central contribution is the sim-to-real story (V4 simulation → QACT quantum-sim transfer → physical hardware transfer).

- Target venue: IEEE Access (Q2). No section drafting, outline, or submission-checklist work has been started yet.

- Suggested first step once hardware data exists: build the results/comparison table across all three regimes (V4 sim, QACT sim-to-sim, hardware sim-to-real) before writing prose.

# 5. Software Fine-Tuning Opportunities

Important framing: V4 Seed 3 is locked and must not be retrained. Everything below is scoped as future-work, to be run as new, separately labeled experiments (e.g. "V5") if you choose to pursue them — for strengthening the paper's discussion section, addressing the OU-regime null result, and improving the QACT sim-to-real narrative. None of it should overwrite existing results.

## 5.1 Addressing the OU-Regime Null Result

V4 beats PID significantly under Gaussian drift but not under Ornstein–Uhlenbeck drift. This is worth investigating rather than leaving as a footnote, since it directly affects how strong a claim the paper can make.

- Domain randomization across drift types: train a variant on a curriculum that mixes Gaussian and OU drift per-episode (rather than a single fixed regime), so the policy is not implicitly Gaussian-specialized.

- Correlated-noise-aware observation: OU drift has memory (it's a mean-reverting process), so feeding the agent a short history of past amplitude readings (e.g. last 4–8 steps) rather than a single-step observation may let it exploit the correlation structure PID cannot use.

- Recurrent policy option: swap the MLP policy for an LSTM/GRU-based PPO policy (RecurrentPPO in sb3-contrib) so the agent carries state across steps — directly relevant since this is a POMDP (no direct frequency access).

- Report OU results with the same rigor as Gaussian: effect size (not just p-value), and confidence interval on the MAE gap, so a non-significant result is presented as "no measurable difference from PID here" rather than implying failure.

## 5.2 Reward Shaping / Degenerate-Policy Guardrails

The known risk with this task is a static policy that sits near resonance without tracking, which can still score a deceptively good MAE. A few reward-side changes can make this failure mode harder to reach without touching the locked reward used for V4.

- Add a small tracking-effort or movement-justified term: reward correct directional movement in response to drift, not just proximity to the setpoint, so a frozen agent is penalized once drift begins.

- Penalize action variance floor: a very-low-variance action stream over a rolling window is itself a signal of a degenerate policy and can be down-weighted or logged as a training-time diagnostic, independent of the trajectory-inspection check already in use.

- Track "time-to-reacquire" after an induced drift step as a secondary metric alongside MAE — it directly measures tracking behavior and is harder to game with a static policy.

## 5.3 PPO Hyperparameter Sweep (for a Future V5)

| Hyperparameter | Current Risk | Suggested Direction |
| --- | --- | --- |
| Entropy coefficient (ent_coef) | Premature convergence to a static/degenerate policy | Small increase or a linear decay schedule (high early, low late) to preserve exploration during the critical early-training window |
| Learning rate | Fixed LR may overshoot once near-optimal behavior is found | Linear or cosine decay schedule instead of a constant rate |
| n_steps / batch_size | Current values tuned for Gaussian regime only | Re-sweep jointly if the OU-curriculum change in 5.1 is adopted, since correlated noise changes the useful rollout length |
| gamma (discount) | May under-weight slow OU mean-reversion dynamics | Try a slightly higher gamma variant specifically for OU-drift training runs |
| clip_range | Standard default; not yet identified as an issue | Leave as-is unless the entropy/LR changes destabilize training |

As with V4, any new sweep should be run across multiple seeds (V4 used Seed 3 as the reported seed) so seed variance is reported alongside the mean, and should go through the same visual-trajectory verification step before trusting the MAE number.

## 5.4 QACT Sim-to-Real Transfer Improvements

QACT v5's large standard deviation (4581 Hz against a 3237 Hz mean) suggests the policy is inconsistent across episodes rather than uniformly mediocre — this is a useful diagnostic, not just noise.

- Break down the 4581 Hz standard deviation by episode: if a subset of episodes drive most of the variance, that points to specific quantum-sim conditions (e.g. certain drift phases or noise realizations) where transfer fails, which is a much stronger paper finding than a single aggregate number.

- Action and observation noise injection during the original classical training (for any future V5) would directly target sim-to-real/sim-to-sim robustness — training with synthetic sensor noise and actuator jitter tends to narrow exactly this kind of variance gap.

- Domain randomization on resonator Q-factor / damping during classical training, so the policy has seen a range of dynamics rather than a single fixed LC response, which should help it generalize into the quantum-simulator dynamics.

- Since QACT is formally stopped, none of this should reopen QACT itself — it should instead inform the training recipe used for the hardware-facing HIL fine-tuning stage described in Section 7 (Phase 4).

## 5.5 Observation / Normalization Hygiene

- VecNormalize running statistics: confirm the saved normalization statistics from V4 training are frozen and shipped alongside the policy weights for any deployment or evaluation — a mismatch here silently degrades performance without changing the model file.

- When Stage 2/5 hardware-in-the-loop data starts arriving, budget time to characterize real ADC/DAC quantization noise so it can be matched in the simulation observation space for future fine-tuning — real 12-bit SPI noise will not exactly match the simulation's assumed noise model.

## 5.6 Evaluation Protocol Hardening

- Continue the established rule: never accept a new MAE number without the paired trajectory plot (agent vs. reference line) — this is already a proven safeguard and should be written into the paper's methodology section as a stated practice, not just an internal habit.

- Add a standard secondary metric set for any future runs: near-resonance fraction (already used), time-to-reacquire after drift, and action-variance floor (Section 5.2) — reporting all three make it much harder for a future degenerate policy to slip through unnoticed.

- Keep the Gemini-implementation cross-check in place: continue diagnosing unauthorized parameter drift (omega_rabi, action_scale, ema_alpha have all recurred before) from trajectory analysis before accepting any new result as valid.

# 6. Hardware Status and Component Reference

## 6.1 Confirmed Bill of Materials

| Component | Role / Notes |
| --- | --- |
| ESP32 | Main controller. VSPI (DAC): SCK=GPIO18, MOSI=GPIO23, CS=GPIO5. HSPI (ADC): SCK=GPIO14, MISO=GPIO12, MOSI=GPIO13, CS=GPIO15 |
| MCP4921 | 12-bit SPI DAC — Wokwi Stage 2, pending |
| MCP3202 | 12-bit SPI ADC — Wokwi Stage 5, pending |
| OPA2134UA | Op-amp, SOIC-8 to DIP-8 adapter required |
| Bourns SRR1280-100M | 10 µH inductor for the LC resonator |
| BAT54-13-F | Schottky diode |
| AMS1117-3.3 (×2) | Separate analog and digital rail regulators — Stage 1 confirmed in LTSpice |
| BB909 / MV209 | Varactor diode — NOT YET ORDERED. Substitute: BB135,115 (NXP). Do not use BB910. |
| 1 µF capacitors (C0G) | PWM RC filter — PREVIOUSLY OUT OF STOCK, recheck sourcing |

## 6.2 Design Rules to Enforce During Build

- C0G dielectric is mandatory for the LP filter and varactor coupling capacitors — X7R is explicitly prohibited due to its voltage/temperature-dependent capacitance.

- 3 cm maximum lead length in the RF path — a daisy-chained run of 400-point breadboards will violate this; plan the physical layout around it before starting the build.

- Keep analog and digital supply rails on their separate AMS1117-3.3 regulators as designed — do not consolidate onto one rail during bring-up.

## 6.3 Open Caveat to Resolve Before Build

Stage 4's LTSpice simulation is complete but flagged with an op-amp supply wiring caveat. Before trusting this stage on hardware, re-derive and re-check the OPA2134UA supply connections against the Stage 4 schematic — this is a five-minute check that should happen before any solder touches the board.

# 7. Hardware Integration Roadmap

| Phase | Focus | Depends On |
| --- | --- | --- |
| Phase 1 | Circuit validation | Wokwi Stages 2 & 5 complete + components procured |
| Phase 2 | System identification | Phase 1 board bring-up and Stage 4 caveat resolved |
| Phase 3 | HIL dataset collection | Phase 2 identified system parameters |
| Phase 4 | Sim-to-real fine-tuning | Phase 3 dataset; informed by Section 5.4 recommendations |
| Phase 5 | Closed-loop deployment | Phase 4 fine-tuned policy validated on hardware |

Current position: pre-Phase 1, blocked on Wokwi Stages 2/5 and the two procurement items in Section 6.1.

# 8. Key Lessons Learned So Far

- MAE alone is not a reliable tracking-quality indicator — a degenerate static policy can score a misleadingly good MAE; visual trajectory inspection (agent vs. reference line) is a required check, not an optional one.

- Unauthorized parameter drift is a recurring risk when a second LLM (Gemini) implements code changes — omega_rabi, action_scale, and ema_alpha have all been silently altered before. Every Gemini-implemented change should be diffed and diagnosed via trajectory analysis before acceptance.

- All completed experiment results are locked under a strict do-not-retrain policy — this report's Section 5 recommendations are explicitly future-work, not modifications to V4 Seed 3.

- On Windows, placing CUDA verification gates at module top-level breaks SubprocVecEnv workers under spawn-based multiprocessing; the fix is moving such checks inside the training function — already applied and confirmed working.

- Wokwi requires raw SPI.h with transfer16() for the MCP4921, since the MCP_DAC external library caused build failures — apply the same raw-SPI approach as the starting point for the MCP3202 ADC stage.

- C0G dielectric is mandatory for RF-adjacent capacitors in this design; X7R is not an acceptable substitute despite being more commonly stocked.

- BB910 is the wrong varactor part number for this design — BB909 or MV209 is correct, with BB135,115 (NXP) as the confirmed local substitute.

# 9. Immediate Next Steps (This Week)

- 1. Re-open the Wokwi workspace and start Stage 2 (MCP4921 DAC) using the raw SPI.h + transfer16() approach already identified.

- 2. Build Stage 5 (MCP3202 ADC) on HSPI once Stage 2 is verified, and confirm no VSPI/HSPI bus contention when both run together.

- 3. Re-check stock for BB909/MV209 (or confirm BB135,115 substitute) and 1 µF C0G capacitors at robu.in; place both orders in parallel with the Wokwi work.

- 4. Re-derive the Stage 4 op-amp supply wiring in LTSpice to close out the outstanding caveat before any physical build begins.

- 5. Decide whether to pursue any Section 5 fine-tuning experiments (e.g. an OU-curriculum V5) as parallel background work, or defer all software work until after hardware Phase 1–3, given V4/QACT are already locked and complete.

- 6. Once Wokwi and procurement are both clear, begin Phase 1 (circuit validation) per the Section 7 roadmap.

End of report.