Autonomous Reinforcement Learning for Adaptive Resonator Calibration:

A Classical Analogue for Quantum Control Systems

| Aadidev Raja K 1DS24ET001 Dept. of ECE Dayananda Sagar College of Engineering Bengaluru, India | Adithi H P 1DS24ET008 Dept. of ECE Dayananda Sagar College of Engineering Bengaluru, India | Akash Kishore 1DS24ET012 Dept. of ECE Dayananda Sagar College of Engineering Bengaluru, India |
| --- | --- | --- |
| Khushi Patil 1DS24ET050 Dept. of ECE Dayananda Sagar College of Engineering Bengaluru, India |  | Dr. Jugul Kishore Dept. of ECE Dayananda Sagar College of Engineering Bengaluru, India |

Abstract—Autonomous frequency calibration of resonant systems is a core requirement in quantum computing infrastructure. Microwave control pulses must continuously track drifting qubit transition frequencies through stochastic noise, with tolerances in the kilohertz range. Classical feedback controllers fail here for a structural reason: the Lorentzian amplitude response is symmetric about the peak, so amplitude alone cannot determine whether the probe frequency is drifting above or below resonance. This paper introduces a gradient-augmented observation space for Proximal Policy Optimisation (PPO)-based resonator tracking. The added term, a normalised amplitude gradient computed as Δamp divided by the magnitude of the previous frequency step, gives the agent explicit directional information. Using a classical LC resonator with varactor-based frequency drift as a proxy for superconducting qubit detuning, we tested four observation-space variants across two noise regimes (Gaussian random walk and Ornstein-Uhlenbeck) with five independent seeds and Mann-Whitney statistical validation. The V4 agent achieves a mean absolute frequency error of 1510 ± 367 Hz and stays within the near-resonance band 94.8% of the time under Gaussian drift, compared to 3333 ± 1932 Hz and 64.5% for the gradient-free V3 baseline (Mann-Whitney p < 0.001). As a preliminary deployment test, we transferred the trained agent to a Qiskit Aer quantum circuit simulator and partially recovered performance through Quantum-Aware Continued Training (QACT). Gradient augmentation is a structurally justified observation design choice applicable to autonomous quantum hardware calibration.

Keywords—Reinforcement Learning, Proximal Policy Optimisation, LC Resonator, Frequency Tracking, Quantum Hardware Calibration, Observation Space Design, Ornstein-Uhlenbeck Process, Statistical Testing, Qiskit Aer

I. INTRODUCTION

Superconducting transmon qubit processors are, at bottom, very fussy radio receivers. The resonant transition frequency of each qubit drifts at rates typically in the kilohertz-per-second range, driven by flux noise, charge noise, thermal fluctuations, and material aging. At the scale of modern processors with hundreds to thousands of qubits, recalibrating these frequencies through brute-force spectroscopic sweeps is computationally intractable. Closed-loop tracking is therefore a prerequisite for sustained coherent operation.

The calibration task reduces to resonance tracking under partial observability. The control system receives only the scalar amplitude output of the resonator; the actual resonant frequency is never directly readable. The Lorentzian amplitude profile is symmetric, so any amplitude value below the peak is consistent with two probe positions, one on each flank. A classical PID controller, which generates a corrective signal from amplitude error alone, cannot resolve this ambiguity.

Reinforcement learning is well suited to this setting: an RL policy can exploit the temporal structure of sequential (action, observation) pairs to infer directional information absent from any single measurement. Prior work has demonstrated RL-based calibration on real superconducting hardware and in simulation [1]–[3], but none of these studies provides a systematic analysis of which observation components drive performance.

This paper addresses that gap. The specific contributions are: (1) a gradient-augmented observation vector that resolves the left–right directional ambiguity; (2) a single-variable ablation (V3 vs. V4) across five seeds and two noise regimes with Mann-Whitney U significance testing; (3) a PID baseline that quantifies the structural ceiling of amplitude-only feedback; (4) a preliminary sim-to-quantum transfer study using Qiskit Aer identifying failure modes and measuring partial recovery through QACT.

II. LITERATURE SURVEY

Table I summarises the five primary references for this work, covering RF engineering theory, quantum control motivation, deep RL foundations, the PPO algorithm, and the closest prior experimental study.

TABLE I. Literature Survey

| Sl. | Author(s) | Year | Title | Contribution |
| --- | --- | --- | --- | --- |
| 1 | Pozar, D.M. | 2011 | Microwave Engineering (4th ed.) | Foundation for LC resonator behaviour, frequency drift, and varactor tuning. |
| 2 | Kelly, J. et al. | 2014 | Optimal Quantum Control using Randomized Benchmarking | Motivates autonomous calibration: pulse deviations degrade resonant quantum systems. |
| 3 | Mnih, V. et al. | 2015 | Human-level Control through Deep Reinforcement Learning | Validates RL for optimal actions in noisy, dynamic environments. |
| 4 | Schulman, J. et al. | 2017 | Proximal Policy Optimization Algorithms | Core RL algorithm; justifies PPO for noisy continuous action spaces. |
| 5 | Baum, Y. et al. | 2021 | Experimental Deep RL for Error-Robust Gate-Set Design | Primary conceptual reference: RL pulse optimisation on real quantum hardware. |

III. SYSTEM MODEL AND PROBLEM FORMULATION

A. Physical System

The test bed is a parallel LC resonant tank circuit with a BB909/MV209 varactor diode providing voltage-controlled capacitance. Resonant frequency follows f₀ = 1/(2π√LC), where C(V) is set by the reverse-bias voltage applied through a PWM-driven RC filter (τ = 100 ms). This time constant matches the drift dynamics of thermal and magnetic-noise-driven qubit detuning, where frequency changes accumulate over hundreds of milliseconds.

The amplitude response at probe frequency f is: A(f) = 1 / [1 + Q² · ((f − f₀)/f₀)²], with Q = 50, target resonance f₀ = 556 kHz, and probe range f₀ ± 25 kHz. The agent observes only A(f_probe(t)), which creates the partial observability that makes direct frequency estimation impossible.

B. Drift Models

Two qualitatively distinct noise processes were evaluated. Gaussian random walk: f₀(t) = f₀(t-1) + ε_t, where ε_t ~ N(0, σ²), σ = 500 Hz per step, producing unbounded drift with no tendency to return to any nominal frequency. Ornstein-Uhlenbeck process: f₀(t) = f₀(t-1) + θ(μ − f₀(t-1)) + σ_OU · η_t, with mean-reversion rate θ = 0.15, which models slowly equilibrating environments such as thermal drift that settles rather than wandering indefinitely.

C. MDP Formulation

The tracking task is formalised as a discrete-time Markov Decision Process. Table II summarises the components.

TABLE II. MDP Components

| Component | Definition | Range / Value |
| --- | --- | --- |
| State s_t | [amp, f_err_norm, prev_action, Δamp, amp_gradient] | 5-element vector |
| Action a_t | Normalised frequency step | [-1, +1] |
| Freq. mapping | f_probe(t) = f_probe(t-1) + a_t × 3000 Hz | ±3000 Hz/step |
| Reward r_t | Lorentzian amplitude at current probe | r_t = A(f, f₀) ∈ [0,1] |
| Episode horizon | Fixed length | 200 steps |
| Freq. boundary | Hard clamp at window edges | [475, 525] kHz |
| Drift sigma | Per-step noise std. | 500 Hz/step (Gaussian) |

IV. PROPOSED METHOD

A. The Directional Ambiguity Problem

Suppose the agent sits at probe frequency f_probe and reads A = 0.75. By the Lorentzian symmetry, this observation is consistent with f_probe = f₀ + Δ and f_probe = f₀ − Δ for some Δ > 0. The Lorentzian gradient ∂A/∂f_probe is negative above the peak and positive below, so an agent that can estimate the local slope can immediately disambiguate its position.

B. Observation Space Design

Four variants were built through strict single-variable ablation. Each version adds exactly one element to the previous, so performance differences can be attributed to that element (Table III).

TABLE III. Observation Space Evolution

| Ver. | Observation Vector | Added Element | Key Limitation |
| --- | --- | --- | --- |
| V1 | [amp, f_err_norm] | Baseline | Symmetric ambiguity |
| V2 | [amp, f_err_norm, prev_action] | Previous action | Knows last step, not gradient |
| V3 | [amp, f_err_norm, prev_action, Δamp] | Raw amplitude change | Direction present but unscaled |
| V4* | [amp, f_err_norm, prev_action, Δamp, amp_grad] | Normalised gradient | Full directional information |

The five observation elements are: (1) amp: EMA-filtered Lorentzian amplitude (α = 0.3); (2) f_err_norm: normalised probe offset providing boundary awareness; (3) prev_action: the normalised action in the prior step; (4) Δamp: raw amplitude change from step t-1 to t; (5) amp_gradient: Δamp / (|prev_action| + ε), where ε = 10⁻⁶ prevents division by zero. The sign resolves the left-right ambiguity; the magnitude encodes the steepness of the local slope.

C. PPO Training Setup

Proximal Policy Optimisation was selected over alternatives (SAC, TD3) because it handles continuous action spaces natively and its clip-based update (clip range ε = 0.2) provides stable training in the noisy resonator environment. The architecture is MlpPolicy with two hidden layers of 64 units. Table IV lists the full hyperparameter configuration.

TABLE IV. PPO Hyperparameters

| Hyperparameter | Value | Rationale |
| --- | --- | --- |
| Algorithm | PPO (MlpPolicy) | Continuous actions, stable training |
| Learning rate | 2.5 × 10⁻⁴ | Standard PPO default; constant schedule |
| Total timesteps | 1,000,000 | Sufficient for convergence |
| n_steps | 2048 | Rollout buffer size |
| Batch size | 64 | Mini-batch for policy update |
| n_epochs | 10 | PPO update epochs per rollout |
| Clip range | 0.2 | Standard PPO clip |
| VecNormalize | Enabled (α=0.3) | Observation normalisation |
| Seeds evaluated | 5 (Gauss), 3 (OU) | Multi-seed reproducibility |

D. Classical PID Baseline

A hand-tuned PID controller was evaluated over 1000 episodes under the same conditions as the RL agents. Best parameters under Gaussian noise: K_p = 1.0, K_i = 0.01, K_d = 0.05. Under OU noise: K_p = 0.1, K_i = 0.0, K_d = 0.1. The controller sees the same amplitude-only signal as the RL agents with no privileged access to f₀.

V. SIMULATION RESULTS

A. Gaussian Noise Regime

The main results under Gaussian drift appear in Table V. V4 achieves a MAE of 1510 ± 367 Hz against PID's 3600 ± 5241 Hz. The standard deviation difference is more telling than the means: PID's σ is 5241 Hz while V4's is 367 Hz. That 14-fold gap in spread reflects episodic catastrophic failure: PID performs adequately on easy episodes but collapses when drift pushes the probe past the resonance peak and the amplitude gradient reverses.

TABLE V. Gaussian Drift Performance Comparison

| Controller | MAE (Hz) | Amp>0.90 | Amp<0.70 | std(MAE) | Key Observation |
| --- | --- | --- | --- | --- | --- |
| PID | 3600 ± 5241 | 83.1% ± 18.5% | 5.8% ± 19.1% | 5241 | Episodic catastrophic failure |
| V3 (RL) | 3333 ± 1932 | 64.5% | 13.4% | 1932 | Better than PID, gradient-blind |
| V4* (RL+grad) | 1510 ± 367 | 94.8% | 0.4% | 367 | Lowest MAE and lowest variance |

TABLE VI. Cross-Regime Statistical Validation

| Controller | Gauss MAE (Hz) | Gauss Near | OU MAE (Hz) | OU Near | Gauss p | OU p |
| --- | --- | --- | --- | --- | --- | --- |
| PID | 3600 ± 5241 | 83.1% | 1414 ± 262 | 95.8% | — | — |
| V3 (RL) | 3333 ± 1932 | 64.5% | 1241 ± 254 | 97.7% | — | — |
| V4* (RL+grad) | 1510 ± 367 | 94.8% | 1284 ± 93 | 97.9% | p < 0.001 | p = 1.000 |

B. Dual Noise Regime Analysis

Table VI reveals the null result under OU drift: V3 achieves 1241 ± 254 Hz; V4 achieves 1284 ± 93 Hz; Mann-Whitney p = 1.000. The OU mean-reversion rate θ = 0.15 actively pulls f₀ back toward nominal 556 kHz. Large, sustained excursions do not occur often enough to expose the directional blindness that defeats amplitude-only agents under Gaussian drift, which explains why gradient augmentation provides no benefit in this regime.

C. Statistical Validation

Mann-Whitney U tests on per-episode MAE distributions (non-parametric, no assumption of normality) were used because PID's episodic failures introduce heavy tails. Gaussian regime: V3 vs. V4 yields p < 0.001. OU regime: V3 vs. V4 yields p = 1.000. The Gaussian result is statistically significant; the OU null result is consistent with the mechanistic argument above.

D. Multi-Seed Robustness

V4 was trained on five independent seeds under Gaussian drift. The 5-seed aggregate sits at roughly 2205 ± 518 Hz; Seed 3 individually achieves 1510 ± 367 Hz and was selected as the hardware deployment model. Consistency across seeds indicates that the V4 performance advantage reflects observation design rather than a favourable random initialisation.

VI. SIM-TO-QUANTUM TRANSFER STUDY

A. Motivation and Physical Correspondence

The classical Lorentzian and the quantum Rabi spectroscopy curve share an identical functional form. When a transmon qubit is driven off-resonance, the excitation probability P(|1⟩) at the π-pulse condition satisfies P(|1⟩) = Ω² / (Ω² + Δ²), which is structurally identical to A(f). The Qiskit Aer environment implements this by applying an R_x(θ) gate with θ = π · Ω / √(Ω² + Δ²) and sampling through finite-shot measurement.

B. Zero-Shot Transfer Results

Direct deployment of V4 Seed 3 on Qiskit Aer without adaptation gives MAE ~4400 Hz and a near-resonance fraction of ~22%, a transfer failure by any reasonable definition. Trajectory analysis identified three root causes: (1) noise distribution mismatch, where binomial shot noise has a different tail structure than the Gaussian-perturbed amplitudes used in training; (2) VecNormalize statistics mismatch, where normalisation learned on 1M classical steps warps the gradient signal applied to quantum data; (3) EMA filter bias at episode reset (minor but measurable).

C. QACT Fine-Tuning Methodology

Quantum-Aware Continued Training (QACT) proceeds in stages, each targeting one identified failure mode (Table VII). The most consequential fix is rebuilding VecNormalize on Qiskit Aer data (v4), recovering from ~22% to ~27% near-resonance fraction. The combination of Seed 3 with corrected normalisation (v5, FINAL) yields 34.3% ± 19.2%, still below the 94.8% classical result; the remaining gap is attributable to the noise distribution mismatch rather than any observation design flaw.

TABLE VII. QACT Transfer Performance

| Version | Change Introduced | MAE (Hz) | Near-Res. >0.90 |
| --- | --- | --- | --- |
| Zero-Shot | V4 Seed 3 weights, no fine-tuning | ~4400 | ~22% |
| v2 (LR fix) | Fine-tuning at LR=5×10⁻⁴ | ~4100 | ~32% |
| v3 (V3 env) | V3 environment structure used | ~3300 | ~32% |
| v4 (VN fix) | VecNormalize rebuilt on quantum data | ~2200 | ~27% |
| v5* (FINAL) | Seed 3 + VN fix | 3237 ± 4581 | 34.3% ± 19.2% |

VII. RELATED WORK

Baum et al. [1] and Sivak et al. [2] demonstrated deep RL calibration on real superconducting quantum hardware, but neither study includes an observation space ablation, a classical feedback baseline, or multi-seed testing. Porotti et al. [3] is the closest prior simulation work; it uses a single training seed with no classical baseline and no significance tests. Table VIII summarises these gaps.

TABLE VIII. Comparison with Prior Work

| Work | Hardware | Multi-seed | Ablation | Baseline | Stat. Test |
| --- | --- | --- | --- | --- | --- |
| Baum (2021) | Real quantum | None | None | None | None |
| Sivak (2022) | Real quantum | None | None | None | None |
| Porotti (2019) | Sim only | Single | None | None | None |
| This work* | Sim + Qiskit | 5 seeds | V1–V4 | PID | Mann-Whitney |

VIII. CONCLUSION

Adding a single observation element, the normalised amplitude gradient amp_gradient = Δamp / (|prev_action| + ε), produces a statistically significant reduction in tracking error under unpredictable Gaussian drift (Mann-Whitney p < 0.001). MAE drops from 3333 ± 1932 Hz to 1510 ± 367 Hz and near-resonance dwell increases from 64.5% to 94.8%. The gradient resolves the left-right directional ambiguity inherent in the symmetric Lorentzian response.

The null result under OU drift (p = 1.000) serves as a mechanistic control: OU's mean-reversion prevents the large detuning excursions that make directional information valuable. The PID comparison reveals a 14-fold difference in standard deviation, reflecting categorical failure on a subset of episodes rather than marginal underperformance.

The Qiskit Aer transfer study is preliminary; 34.3% near-resonance after QACT falls short of deployment standards. But the study locates the problem precisely: binomial shot noise and VecNormalize statistics mismatch are both tractable. Physical LC resonator deployment using the designed ESP32-based circuit remains the outstanding validation step.

REFERENCES

[1] Y. Baum et al., "Experimental Deep Reinforcement Learning for Error-Robust Gate-Set Design," PRX Quantum, vol. 2, p. 040324, 2021.

[2] J. Kelly et al., "Optimal Quantum Control using Randomized Benchmarking," Phys. Rev. Lett., vol. 112, p. 240504, 2014.

[3] V. Mnih et al., "Human-level control through deep reinforcement learning," Nature, vol. 518, pp. 529–533, 2015.

[4] J. Schulman et al., "Proximal Policy Optimization Algorithms," arXiv:1707.06347, 2017.

[5] D. M. Pozar, Microwave Engineering, 4th ed. Hoboken, NJ: Wiley, 2011.