Autonomous Resonator Control System

Complete Simulation & Breadboard Integration Guide

Wokwi: DAC + ADC + ESP32 SPI  |  LTspice Reference Summary  |  Full 5-Stage Breadboard Build

DSCE — Electronics & Telecommunication — 2025-26

# PART A — Complete Project Stage Map

Your project has exactly five circuit stages. The table below shows which stages are already simulated and which this document completes.

| Stage | Circuit | Simulation Tool | Status | Gate to Unlock |
| --- | --- | --- | --- | --- |
| 1 | AMS1117 Power Supply | LTspice .op | DONE ✅ | Stage 2 & 4 can be powered |
| 2 | MCP4921 DAC + ESP32 VSPI | Wokwi | THIS DOC ▶ | Stage 3 receives waveform |
| 3 | LC Resonator + Varactor | LTspice AC sweep | DONE ✅ | Stage 4 receives RF signal |
| 4 | Envelope Detector | LTspice transient | DONE ✅ | Stage 5 receives DC amplitude |
| 5 | MCP3202 ADC + ESP32 HSPI | Wokwi | THIS DOC ▶ | Firmware reads amplitude |

▶  There are NO additional simulation stages beyond these five. Once Wokwi verifies the DAC and ADC SPI firmware, all simulation work is complete and breadboard construction begins.

# PART B — Wokwi Simulation: Stage 2 — MCP4921 DAC + ESP32

Goal: Prove that the ESP32 firmware correctly sends 16-bit SPI frames to the MCP4921, producing a staircase/sine waveform on VOUT that a scope confirms. This replaces the need for a physical DAC during firmware development.

## B.1 — Wokwi Project Setup

- Open wokwi.com in your browser. Click New Project.

- Select ESP32 from the list of boards. An empty project opens with a sketch editor on the left and a diagram panel on the right.

- Click the diagram.json tab at the top of the diagram panel. Delete all existing content and paste the complete JSON from Section B.3 below.

- Click the sketch.ino tab and paste the complete firmware from Section B.4 below.

- Click the green Play (Run) button. The simulation starts immediately — no compile step needed.

## B.2 — Component Connections: MCP4921 DAC

The MCP4921 is an 8-pin DIP SPI DAC. All connections use the VSPI bus on the ESP32.

| MCP4921 Pin | Pin Name | Connects To | Wire Colour | Series/Parallel | Critical Note |
| --- | --- | --- | --- | --- | --- |
| Pin 1 | VDD | ESP32 3V3 | Red | — | Power. Must be stable 3.3V. |
| Pin 2 | CS | ESP32 GPIO5 | Green | — | Active-LOW chip select. Must be HIGH when idle. |
| Pin 3 | SCK | ESP32 GPIO18 | Yellow | — | SPI clock. VSPI default clock pin. |
| Pin 4 | SDI | ESP32 GPIO23 | Blue | — | MOSI. Data flows ESP32 → DAC only. |
| Pin 5 | LDAC | GND (direct) | Black | — | Tie permanently LOW. Output updates on every rising CS edge. |
| Pin 6 | VOUT | Oscilloscope CH1 + 200Ω to GND | Orange | Series: VOUT → 200Ω → GND | This is your DAC output. The 200Ω is the termination resistor. |
| Pin 7 | AVSS | GND | Black | — | Analog ground. Same net as digital GND in this design. |
| Pin 8 | VREF | ESP32 3V3 | Red | — | Reference = supply. Full-scale output = 3.3V at code 0xFFF. |

▶  LDAC must be tied directly to GND — do not leave it floating. A floating LDAC means the DAC latches asynchronously and the output may glitch between SPI frames.

## B.3 — diagram.json (Paste this exactly into Wokwi)

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stage 2 DAC",

"editor": "wokwi",

"parts": [

{ "type": "wokwi-esp32-devkit-v1", "id": "esp",

"top": 0, "left": 0, "attrs": {} },

{ "type": "wokwi-mcp4921", "id": "dac1",

"top": 60, "left": 380, "attrs": {} },

{ "type": "wokwi-resistor", "id": "r_term",

"top": 160, "left": 560, "attrs": { "value": "200" } },

{ "type": "wokwi-scope", "id": "sc1",

"top": 280, "left": 350, "attrs": {} },

{ "type": "wokwi-logic-analyzer", "id": "la1",

"top": 450, "left": 0, "attrs": {} }

],

"connections": [

["esp:3V3",    "dac1:VDD",  "red",    []],

["esp:3V3",    "dac1:VREF", "red",    []],

["esp:GND.1",  "dac1:GND",  "black",  []],

["esp:GND.2",  "dac1:LDAC", "black",  []],

["esp:18",     "dac1:CLK",  "yellow", []],

["esp:23",     "dac1:SDI",  "blue",   []],

["esp:5",      "dac1:CS",   "green",  []],

["dac1:VOUT",  "r_term:1",  "orange", []],

["r_term:2",   "esp:GND.3", "black",  []],

["dac1:VOUT",  "sc1:CH1",   "orange", []],

["esp:18",     "la1:D0",    "yellow", []],

["esp:23",     "la1:D1",    "blue",   []],

["esp:5",      "la1:D2",    "green",  []]

]

}

## B.4 — Arduino Firmware: DAC Waveform Generator

▶  This sketch outputs a 256-step sine wave through the MCP4921 continuously. The Serial Monitor prints the current DAC code every 100ms so you can confirm SPI is working even without a scope.

#include <SPI.h>

#include <math.h>

// ── Pin Definitions: VSPI Bus (DAC) ─────────────────────

#define DAC_CS_PIN   5    // GPIO5  → MCP4921 Pin 2 (CS)

#define SPI_SCK_PIN  18   // GPIO18 → MCP4921 Pin 3 (SCK)

#define SPI_MOSI_PIN 23   // GPIO23 → MCP4921 Pin 4 (SDI)

// ── MCP4921 Configuration Word ───────────────────────────

// Bit 15: DACA=0 (channel A)

// Bit 14: BUF=0 (unbuffered VREF)

// Bit 13: ~GA=1 (1x gain, VOUT max = VREF = 3.3V)

// Bit 12: ~SHDN=1 (output active)

// Bits 11-0: 12-bit data value

#define MCP4921_CONFIG 0x3000

SPIClass vspi(VSPI);

// ── Write one 12-bit code to MCP4921 ─────────────────────

void dac_write(uint16_t code) {

code &= 0x0FFF;                       // clamp to 12 bits

uint16_t frame = MCP4921_CONFIG | code;

vspi.beginTransaction(SPISettings(8000000, MSBFIRST, SPI_MODE0));

digitalWrite(DAC_CS_PIN, LOW);         // assert CS

vspi.transfer((frame >> 8) & 0xFF);   // MSB first

vspi.transfer(frame & 0xFF);          // LSB

digitalWrite(DAC_CS_PIN, HIGH);        // latch output

vspi.endTransaction();

}

// ── Pre-compute 256-point sine lookup table ───────────────

uint16_t sine_table[256];

void build_sine_table() {

for (int i = 0; i < 256; i++) {

// Range: 200 to 3895 (avoids rail clipping)

sine_table[i] = (uint16_t)(2048 + 1800 * sin(i * 2.0 * M_PI / 256.0));

}

}

void setup() {

Serial.begin(921600);

Serial.println("ARCS_DAC_INIT");

pinMode(DAC_CS_PIN, OUTPUT);

digitalWrite(DAC_CS_PIN, HIGH);  // CS idle HIGH

vspi.begin(SPI_SCK_PIN, -1, SPI_MOSI_PIN, DAC_CS_PIN);

build_sine_table();

Serial.println("ARCS_DAC_READY — outputting 256-step sine");

}

uint8_t phase = 0;

uint32_t last_print = 0;

void loop() {

// Output one sine sample per loop iteration

// At 8MHz SPI: each frame takes ~2.5us → ~390 kHz update rate

// Phase increment of 1 per loop gives ~1.5 kHz effective sine

// (Real 556 kHz would need hardware timer — see note below)

dac_write(sine_table[phase]);

phase++;  // wraps at 256 automatically (uint8_t)

// Print current code every 100ms for Serial Monitor verification

if (millis() - last_print > 100) {

Serial.printf("DAC code: %4d  VOUT_est: %.4f V\n",

sine_table[(phase-1)&0xFF],

sine_table[(phase-1)&0xFF] * 3.3f / 4095.0f);

last_print = millis();

}

}

▶  Note on 556 kHz: Wokwi cannot simulate 556 kHz waveforms accurately because its simulation timestep is too coarse for RF signals. The sine table above runs at ~1.5 kHz which is sufficient to verify SPI frame correctness. The 556 kHz timing is verified by the LC resonator LTspice simulation which has already passed. On real hardware the ESP32 runs the full 556 kHz using a timer ISR.

## B.5 — What to Verify in Wokwi (DAC)

| # | What to Check | Where to Look | Pass Criteria |
| --- | --- | --- | --- |
| 1 | Serial Monitor prints ARCS_DAC_READY | Serial Monitor tab | No error messages. Code values cycle between 248 and 3848. |
| 2 | Oscilloscope shows sine waveform | Scope widget | Smooth sine wave visible on CH1. Peak ~2.75V, trough ~0.65V. |
| 3 | Logic Analyser: 2 bytes per CS pulse | Logic Analyser tab | D2 (CS) goes LOW, exactly 16 clock pulses on D0, then CS HIGH. |
| 4 | First nibble of D1 (MOSI) = 0011 | Logic Analyser decode | Config bits: DACA=0, BUF=0, GA=1, SHDN=1. Always 0x3xxx. |
| 5 | VOUT_est tracks sine_table code | Serial Monitor | e.g. code 2048 → VOUT_est 1.6500V (exactly half of 3.3V). |

# PART C — Wokwi Simulation: Stage 5 — MCP3202 ADC + ESP32

Goal: Prove that the ESP32 firmware correctly reads 12-bit amplitude codes from the MCP3202 via HSPI, applies 64-sample averaging, normalises to [0,1], and prints MEASURE values to Serial. A potentiometer on CH0 simulates the envelope detector output.

## C.1 — Component Connections: MCP3202 ADC

| MCP3202 Pin | Pin Name | Connects To | Wire Colour | Series/Parallel | Critical Note |
| --- | --- | --- | --- | --- | --- |
| Pin 1 | CS/SHDN | ESP32 GPIO15 | Green | — | Active-LOW chip select. HSPI bus. |
| Pin 2 | CH0 | Potentiometer wiper | Orange | — | Analog input. 0–3.3V = 0–4095 codes. |
| Pin 3 | CH1 | GND via 10kΩ | Grey | Series: CH1→10k→GND | Unused channel. Pull to GND to prevent floating noise. |
| Pin 4 | VSS | GND | Black | — | Ground. |
| Pin 5 | CLK | ESP32 GPIO14 | Yellow | — | HSPI clock pin. |
| Pin 6 | Din | ESP32 GPIO13 | Blue | — | MOSI. ESP32 sends start/config bits. |
| Pin 7 | Dout | ESP32 GPIO12 | Purple | — | MISO. ADC sends result bits back. |
| Pin 8 | VDD/VREF | ESP32 3V3 | Red | — | Both power and voltage reference. Must be same rail as envelope detector OPA2134 ANALOG_3V3. |

▶  In hardware: Pin 8 (VDD/VREF) must connect to the ANALOG_3V3 rail (AMS1117 analog output), NOT the digital rail. Using the digital rail introduces SPI switching noise into the ADC reference, degrading accuracy by 2-4 LSB. In Wokwi both rails are ideal so it makes no difference in simulation.

## C.2 — diagram.json (Paste this for the ADC simulation)

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stage 5 ADC",

"editor": "wokwi",

"parts": [

{ "type": "wokwi-esp32-devkit-v1", "id": "esp",

"top": 0, "left": 0, "attrs": {} },

{ "type": "wokwi-mcp3202", "id": "adc1",

"top": 60, "left": 380, "attrs": {} },

{ "type": "wokwi-slide-potentiometer", "id": "pot1",

"top": 60, "left": 580,

"attrs": { "value": "0.5" } },

{ "type": "wokwi-resistor", "id": "r_ch1",

"top": 160, "left": 440,

"attrs": { "value": "10000" } },

{ "type": "wokwi-logic-analyzer", "id": "la1",

"top": 400, "left": 0, "attrs": {} }

],

"connections": [

["esp:3V3",   "adc1:VDD",  "red",    []],

["esp:GND.1", "adc1:VSS",  "black",  []],

["esp:15",    "adc1:CS",   "green",  []],

["esp:14",    "adc1:CLK",  "yellow", []],

["esp:13",    "adc1:DIN",  "blue",   []],

["esp:12",    "adc1:DOUT", "purple", []],

["pot1:SIG",  "adc1:CH0",  "orange", []],

["pot1:VCC",  "esp:3V3",   "red",    []],

["pot1:GND",  "esp:GND.2", "black",  []],

["adc1:CH1",  "r_ch1:1",   "grey",   []],

["r_ch1:2",   "esp:GND.3", "black",  []],

["esp:14",    "la1:D0",    "yellow", []],

["esp:13",    "la1:D1",    "blue",   []],

["esp:12",    "la1:D2",    "purple", []],

["esp:15",    "la1:D3",    "green",  []]

]

}

## C.3 — Arduino Firmware: ADC Reader with Averaging

#include <SPI.h>

// ── Pin Definitions: HSPI Bus (ADC) ─────────────────────

#define ADC_CS_PIN    15   // GPIO15 → MCP3202 Pin 1 (CS)

#define HSPI_SCK_PIN  14   // GPIO14 → MCP3202 Pin 5 (CLK)

#define HSPI_MISO_PIN 12   // GPIO12 → MCP3202 Pin 7 (Dout)

#define HSPI_MOSI_PIN 13   // GPIO13 → MCP3202 Pin 6 (Din)

#define ADC_SAMPLES   64   // oversampling count (reduces noise by sqrt(64)=8x)

#define ADC_MAX       4095 // MCP3202 is 12-bit: 0-4095

SPIClass hspi(HSPI);

// ── Read CH0 from MCP3202 ────────────────────────────────

// Protocol (MCP3202 datasheet Fig 6-1):

//   Byte 1: 0x01  (start bit)

//   Byte 2: 0x80  (SGL=1 CH0, ODD=0)

//   Byte 3: 0x00  (dummy — clocks out result)

// Result: bits 11:8 in byte2[3:0], bits 7:0 in byte3[7:0]

uint16_t adc_read_ch0() {

hspi.beginTransaction(SPISettings(1000000, MSBFIRST, SPI_MODE0));

digitalWrite(ADC_CS_PIN, LOW);

hspi.transfer(0x01);              // start bit

uint8_t hi = hspi.transfer(0x80); // single-ended CH0

uint8_t lo = hspi.transfer(0x00); // clock out lower bits

digitalWrite(ADC_CS_PIN, HIGH);

hspi.endTransaction();

return ((hi & 0x0F) << 8) | lo;  // 12-bit result

}

// ── 64-sample average ────────────────────────────────────

float adc_read_averaged() {

uint32_t acc = 0;

for (int i = 0; i < ADC_SAMPLES; i++) {

acc += adc_read_ch0();

delayMicroseconds(50);   // 50us between samples = ~20kSPS

}

float raw_avg = acc / (float)ADC_SAMPLES;

return raw_avg / ADC_MAX;   // normalise to [0.0, 1.0]

}

void setup() {

Serial.begin(921600);

Serial.println("ARCS_ADC_INIT");

pinMode(ADC_CS_PIN, OUTPUT);

digitalWrite(ADC_CS_PIN, HIGH);   // CS idle HIGH

hspi.begin(HSPI_SCK_PIN, HSPI_MISO_PIN, HSPI_MOSI_PIN, ADC_CS_PIN);

Serial.println("ARCS_ADC_READY");

Serial.println("Move the potentiometer slider to change amplitude.");

}

void loop() {

float amplitude = adc_read_averaged();

// Format: Python parser expects this exact string

Serial.printf("MEASURE %.6f\n", amplitude);

// Also print voltage estimate for debugging

Serial.printf("  -> raw_V: %.4f V   (pot at %.1f%%)\n",

amplitude * 3.3f, amplitude * 100.0f);

delay(50);   // 50ms between measurement cycles = 20 Hz read rate

}

## C.4 — What to Verify in Wokwi (ADC)

| # | What to Check | Where to Look | Pass Criteria |
| --- | --- | --- | --- |
| 1 | ARCS_ADC_READY prints on startup | Serial Monitor | No errors. Prints within 2 seconds of pressing Play. |
| 2 | MEASURE values print at ~20Hz | Serial Monitor | Continuous stream of MEASURE 0.xxxxxx lines. |
| 3 | Pot at 0% → MEASURE near 0.0 | Slide pot to bottom | MEASURE should be 0.000000 to 0.005000. |
| 4 | Pot at 50% → MEASURE near 0.5 | Slide pot to middle | MEASURE should be 0.490000 to 0.510000. |
| 5 | Pot at 100% → MEASURE near 1.0 | Slide pot to top | MEASURE should be 0.995000 to 1.000000. |
| 6 | Logic Analyser: 3 bytes per CS pulse | LA tab, decode as SPI | D3 (CS) LOW → 24 clock pulses on D0 → D2 carries result → CS HIGH. |
| 7 | MOSI byte 1 = 0x01 | LA MOSI decode | First byte always 0x01 (start bit). If 0x00, check transfer order. |
| 8 | MOSI byte 2 = 0x80 | LA MOSI decode | Second byte = 0x80 = CH0 single-ended. If 0xD0, CH1 is selected — wrong. |

▶  If MEASURE values are always 0.000 regardless of pot position: (1) check pot:VCC is connected to 3V3, not GND. (2) Check pot:SIG connects to adc1:CH0, not adc1:CH1. (3) Verify ADC_CS_PIN is 15, not 5 — pin 5 is the DAC CS on the other bus.

# PART D — Combined DAC + ADC Simulation (Both Buses Together)

This simulation runs Stage 2 and Stage 5 simultaneously on the same ESP32. It verifies that VSPI (DAC) and HSPI (ADC) operate independently without interference. This is the final firmware validation before hardware build.

## D.1 — Combined diagram.json

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stage 2+5 Combined",

"editor": "wokwi",

"parts": [

{ "type": "wokwi-esp32-devkit-v1", "id": "esp",

"top": 0, "left": 0, "attrs": {} },

{ "type": "wokwi-mcp4921", "id": "dac1",

"top": 60, "left": 380, "attrs": {} },

{ "type": "wokwi-mcp3202", "id": "adc1",

"top": 220, "left": 380, "attrs": {} },

{ "type": "wokwi-resistor", "id": "r_term",

"top": 120, "left": 560, "attrs": { "value": "200" } },

{ "type": "wokwi-slide-potentiometer", "id": "pot1",

"top": 220, "left": 580, "attrs": { "value": "0.5" } },

{ "type": "wokwi-resistor", "id": "r_ch1",

"top": 300, "left": 440, "attrs": { "value": "10000" } },

{ "type": "wokwi-scope", "id": "sc1",

"top": 400, "left": 380, "attrs": {} },

{ "type": "wokwi-logic-analyzer", "id": "la1",

"top": 500, "left": 0, "attrs": {} }

],

"connections": [

["esp:3V3",   "dac1:VDD",  "red",    []],

["esp:3V3",   "dac1:VREF", "red",    []],

["esp:GND.1", "dac1:GND",  "black",  []],

["esp:GND.2", "dac1:LDAC", "black",  []],

["esp:18",    "dac1:CLK",  "yellow", []],

["esp:23",    "dac1:SDI",  "blue",   []],

["esp:5",     "dac1:CS",   "green",  []],

["dac1:VOUT", "r_term:1",  "orange", []],

["r_term:2",  "esp:GND.3", "black",  []],

["dac1:VOUT", "sc1:CH1",   "orange", []],

["esp:3V3",   "adc1:VDD",  "red",    []],

["esp:GND.4", "adc1:VSS",  "black",  []],

["esp:15",    "adc1:CS",   "green",  []],

["esp:14",    "adc1:CLK",  "yellow", []],

["esp:13",    "adc1:DIN",  "blue",   []],

["esp:12",    "adc1:DOUT", "purple", []],

["pot1:SIG",  "adc1:CH0",  "orange", []],

["pot1:VCC",  "esp:3V3",   "red",    []],

["pot1:GND",  "esp:GND.5", "black",  []],

["adc1:CH1",  "r_ch1:1",   "grey",   []],

["r_ch1:2",   "esp:GND.6", "black",  []],

["esp:18",    "la1:D0",    "yellow", []],

["esp:23",    "la1:D1",    "blue",   []],

["esp:5",     "la1:D2",    "green",  []],

["esp:14",    "la1:D3",    "yellow", []],

["esp:13",    "la1:D4",    "blue",   []],

["esp:12",    "la1:D5",    "purple", []],

["esp:15",    "la1:D6",    "green",  []]

]

}

## D.2 — Combined Firmware (Both Buses)

#include <SPI.h>

#include <math.h>

// ── VSPI Bus: DAC ────────────────────────────────────────

#define DAC_CS   5

#define VSPI_SCK 18

#define VSPI_MOSI 23

SPIClass vspi(VSPI);

// ── HSPI Bus: ADC ────────────────────────────────────────

#define ADC_CS    15

#define HSPI_SCK  14

#define HSPI_MISO 12

#define HSPI_MOSI 13

SPIClass hspi(HSPI);

// ── Sine lookup (256 points) ─────────────────────────────

uint16_t sine_lut[256];

void build_lut() {

for(int i=0;i<256;i++)

sine_lut[i]=(uint16_t)(2048+1800*sin(i*2.0*M_PI/256.0));

}

// ── DAC write ────────────────────────────────────────────

void dac_write(uint16_t code) {

uint16_t f = 0x3000 | (code & 0x0FFF);

vspi.beginTransaction(SPISettings(8000000,MSBFIRST,SPI_MODE0));

digitalWrite(DAC_CS, LOW);

vspi.transfer((f>>8)&0xFF);

vspi.transfer(f&0xFF);

digitalWrite(DAC_CS, HIGH);

vspi.endTransaction();

}

// ── ADC read CH0 (single sample) ─────────────────────────

uint16_t adc_read() {

hspi.beginTransaction(SPISettings(1000000,MSBFIRST,SPI_MODE0));

digitalWrite(ADC_CS, LOW);

hspi.transfer(0x01);

uint8_t hi = hspi.transfer(0x80);

uint8_t lo = hspi.transfer(0x00);

digitalWrite(ADC_CS, HIGH);

hspi.endTransaction();

return ((hi&0x0F)<<8)|lo;

}

// ── ADC averaged read ────────────────────────────────────

float adc_averaged(int n=64) {

uint32_t acc=0;

for(int i=0;i<n;i++){acc+=adc_read();delayMicroseconds(50);}

return (acc/(float)n)/4095.0f;

}

void setup() {

Serial.begin(921600);

pinMode(DAC_CS,OUTPUT); digitalWrite(DAC_CS,HIGH);

pinMode(ADC_CS,OUTPUT); digitalWrite(ADC_CS,HIGH);

vspi.begin(VSPI_SCK,-1,VSPI_MOSI,DAC_CS);

hspi.begin(HSPI_SCK,HSPI_MISO,HSPI_MOSI,ADC_CS);

build_lut();

Serial.println("ARCS_INIT_OK — DAC+ADC combined");

}

uint8_t phase = 0;

uint32_t last_meas = 0;

void loop() {

// DAC: output sine continuously (every loop = one phase step)

dac_write(sine_lut[phase++]);  // phase wraps at 256

// ADC: sample every 50ms (non-blocking check)

if (millis() - last_meas >= 50) {

float amp = adc_averaged(64);

Serial.printf("MEASURE %.6f\n", amp);

last_meas = millis();

}

}

▶  Bus isolation check: In the logic analyser, enable SPI decoding on channels D0/D1/D2 (VSPI) and D3/D4/D5/D6 (HSPI) simultaneously. Confirm that no VSPI clock edge appears during an HSPI frame and vice versa. If they overlap, the firmware has a re-entrancy bug.

# PART E — Full Breadboard Integration: All 5 Stages

Build the stages in strict order. Each stage must pass its verification test before the next is wired. The MB102 830-point breadboard is used throughout.

▶  Before starting any physical build: (1) All 5 simulations must be complete and passing. (2) Varactor BB909 or MV209 must be in hand. (3) OPA2134UA must be soldered onto its SOIC-8 adapter. (4) SMD inductor (SRR1280) must have berg strip leads soldered.

## E.1 — Breadboard Layout Plan

The 830-point MB102 board is divided into 5 zones. Use the coordinate system: columns a-j, rows 1-63 (top to bottom).

| Zone | Rows | Stage | Key ICs |
| --- | --- | --- | --- |
| 1 | 1 – 12 | Power Supply | AMS1117 U1 (Analog), AMS1117 U2 (Digital), bulk caps |
| 2 | 14 – 28 | DAC + ESP32 SPI | MCP4921 U4, termination R1, coupling C6, PWM RC filter |
| 3 | 30 – 42 | LC Resonator | L1 inductor, C8 tank cap, BB909 varactor, bias network |
| 4 | 44 – 55 | Envelope Detector | BAT54 D2, C_HOLD, R_LP, C_LPF, OPA2134 U5 |
| 5 | 57 – 63 | ADC | MCP3202 U6, decoupling cap |
| — | Off-board | ESP32 DevKit | Connected by jumper wires to Zones 2 and 5 |

## E.2 — Stage 1: Power Supply (Build First)

Two separate AMS1117-3.3 regulators supply independent analog and digital 3.3V rails. This isolation prevents DAC/ADC switching noise from corrupting the LC resonator measurement.

| Connection | From | To | Type | Note |
| --- | --- | --- | --- | --- |
| U1 IN | 5V (USB via MB102 PSU) | AMS1117 pin 1 | Wire | Input must be > 4.75V |
| U1 GND | AMS1117 pin 2 | GND rail | Wire | SOT-223: centre pin = GND |
| U1 OUT | AMS1117 pin 3 | Row 5 col a-c (ANALOG_3V3) | Wire | Label this row with tape |
| C1 | ANALOG_3V3 row | GND rail | In parallel | 10µF electrolytic, + to ANALOG_3V3 |
| C2 | ANALOG_3V3 row | GND rail | In parallel | 100nF X7R ceramic, decoupling |
| U2 IN | 5V (MB102 PSU) | AMS1117 pin 1 | Wire | Second regulator, separate area |
| U2 GND | AMS1117 pin 2 | GND rail | Wire |  |
| U2 OUT | AMS1117 pin 3 | Row 10 col a-c (DIGITAL_3V3) | Wire | Label with tape |
| C3 | DIGITAL_3V3 row | GND rail | In parallel | 10µF electrolytic |
| C4 | DIGITAL_3V3 row | GND rail | In parallel | 100nF X7R ceramic |

▶  Verification before Stage 2: Multimeter DC between ANALOG_3V3 and GND → must read 3.28–3.32V. Multimeter DC between DIGITAL_3V3 and GND → must read 3.28–3.32V. Both stable with no flicker.

## E.3 — Stage 2: MCP4921 DAC (After Stage 1 passes)

| Connection | From | To | Type | Note |
| --- | --- | --- | --- | --- |
| U4 placed | Rows 14-18 | Straddles centre gap | DIP-8 | Pin 1 top-left |
| Pin 1 VDD | MCP4921 pin 1 | DIGITAL_3V3 row | Wire |  |
| Pin 2 CS | MCP4921 pin 2 | ESP32 GPIO5 (jumper) | Wire | Active-LOW |
| Pin 3 SCK | MCP4921 pin 3 | ESP32 GPIO18 (jumper) | Wire |  |
| Pin 4 SDI | MCP4921 pin 4 | ESP32 GPIO23 (jumper) | Wire |  |
| Pin 5 LDAC | MCP4921 pin 5 | GND rail | Wire | Direct short to GND. No resistor. |
| R1 200Ω | MCP4921 pin 6 (VOUT) | R1 right end = node DAC_OUT | Series | Termination resistor |
| C6 100nF | DAC_OUT node | Node RESONATOR_IN | Series | AC coupling. Right end feeds Stage 3. |
| C5 100nF | MCP4921 pin 1 | GND rail | In parallel | Decoupling. Place within 3mm of pin. |
| Pin 7 AVSS | MCP4921 pin 7 | GND rail | Wire |  |
| Pin 8 VREF | MCP4921 pin 8 | DIGITAL_3V3 row | Wire |  |
| R2 10kΩ | ESP32 GPIO25 | Node VARACTOR_BIAS | Series | PWM drift resistor |
| C7 1µF | VARACTOR_BIAS node | GND rail | In parallel | PWM RC filter cap. SMD on berg pins. |

▶  Verification: Flash combined DAC+ADC firmware. Oscilloscope probe on R1 right end (DAC_OUT): sine waveform visible. Serial Monitor prints DAC code values cycling. No SPI errors.

## E.4 — Stage 3: LC Resonator (After Stage 2 passes)

| Connection | From | To | Type | Note |
| --- | --- | --- | --- | --- |
| L1 placed | Rows 32-33 | Straddles centre gap | SMD + berg pins | Bourns SRR1280. One terminal to RESONATOR_IN. |
| L1 terminal 1 | L1 pin A | RESONATOR_IN node | Wire | RESONATOR_IN is where C6 right end terminates |
| L1 terminal 2 | L1 pin B | GND rail | Wire | Inductor returns to GND |
| C8 8.2nF | RESONATOR_IN node | GND rail | In parallel with L1 | Tank capacitor. C0G 0805 on berg pins. |
| C10 100pF | RESONATOR_IN node | Node VARACTOR_AC | Series | Varactor AC coupling cap. Right end feeds varactor. |
| D_VAR BB909 | VARACTOR_AC node | Node VARACTOR_ANODE | Series | Cathode to VARACTOR_AC. Anode toward VARACTOR_ANODE. |
| R3 100kΩ | VARACTOR_ANODE node | VARACTOR_BIAS node | Series | Bias isolation. Connects varactor to PWM RC output. |
| R4 10kΩ | RESONATOR_IN node | Node DETECTOR_IN | Series | Output tap. Prevents detector loading the tank. |

▶  Verification: Sweep function generator from 450–650 kHz through RESONATOR_IN. Oscilloscope probe on DETECTOR_IN. A clear Lorentzian amplitude peak must appear near 556 kHz. Record this as your real f0. Varactor bias sweep must shift the peak frequency.

## E.5 — Stage 4: Envelope Detector (After Stage 3 passes)

| Connection | From | To | Type | Note |
| --- | --- | --- | --- | --- |
| D2 BAT54 | DETECTOR_IN node (anode) | Node RECT_OUT (cathode) | Series | SOT-23 on 0805 adapter. Anode left, cathode right. |
| C_HOLD 100nF | RECT_OUT node | GND rail | In parallel | 100nF 630V polyester film. NOT ceramic. |
| R_LP 33kΩ | RECT_OUT node | Node LPF_OUT | Series | ±1% metal film. Sets LP filter cutoff. |
| C_LPF 100nF | LPF_OUT node | GND rail | In parallel | Film cap ONLY. No X7R here. fc = 48 Hz. |
| U5 OPA2134 | Rows 50-52 | Straddles centre gap | SOIC-8 adapter | Pre-soldered on adapter before inserting. |
| U5 IN+ (pin 3) | LPF_OUT node | OPA2134 pin 3 | Wire | Non-inverting input = LP filter output |
| U5 IN- (pin 2) | OPA2134 pin 6 (OUT) | OPA2134 pin 2 | Short wire | Unity-gain feedback. Critical. |
| U5 OUT (pin 6) | OPA2134 pin 6 | Node ENVELOPE_OUT | Wire | Feeds Stage 5 ADC CH0 |
| U5 V+ (pin 7) | OPA2134 pin 7 | ANALOG_3V3 row | Wire | Analog rail. NOT digital. |
| U5 V- (pin 4) | OPA2134 pin 4 | GND rail | Wire |  |
| C13 100nF | OPA2134 pin 7 | GND rail | In parallel | Decoupling. Within 5mm of pin 7. |

▶  Verification: Drive RESONATOR_IN at real f0. Multimeter DC on ENVELOPE_OUT: must read 3.0–3.2V stable. Drive at f0 ±50 kHz: voltage must drop measurably. Oscilloscope on ENVELOPE_OUT: flat DC line with no oscillation.

## E.6 — Stage 5: MCP3202 ADC (After Stage 4 passes)

| Connection | From | To | Type | Note |
| --- | --- | --- | --- | --- |
| U6 placed | Rows 58-62 | Straddles centre gap | DIP-8 | Pin 1 top-left |
| Pin 1 CS | MCP3202 pin 1 | ESP32 GPIO15 (jumper) | Wire | HSPI bus. Different from DAC CS on GPIO5. |
| Pin 2 CH0 | MCP3202 pin 2 | ENVELOPE_OUT node | Wire | ADC reads envelope detector output |
| Pin 3 CH1 | MCP3202 pin 3 | GND via 10kΩ R_CH1 | Series | Unused channel tied to GND |
| Pin 4 VSS | MCP3202 pin 4 | GND rail | Wire |  |
| Pin 5 CLK | MCP3202 pin 5 | ESP32 GPIO14 (jumper) | Wire | HSPI clock |
| Pin 6 Din | MCP3202 pin 6 | ESP32 GPIO13 (jumper) | Wire | MOSI from ESP32 |
| Pin 7 Dout | MCP3202 pin 7 | ESP32 GPIO12 (jumper) | Wire | MISO to ESP32 |
| Pin 8 VDD/VREF | MCP3202 pin 8 | ANALOG_3V3 row | Wire | Both VDD and VREF on ANALOG rail |
| C14 100nF | MCP3202 pin 8 | GND rail | In parallel | Decoupling within 3mm of pin 8. |

▶  Verification: Serial Monitor prints MEASURE values near 1.0 when driving resonator at f0. Values drop toward 0.5 when driving at f0 ±50 kHz. Standard deviation of 20 consecutive readings < 0.01. If > 0.01, increase ADC_SAMPLES to 128.

## E.7 — ESP32 Jumper Wire Map (All 5 Stages Combined)

| ESP32 GPIO | Signal | Destination | Colour | Bus |
| --- | --- | --- | --- | --- |
| GPIO5 | VSPI_CS_DAC | MCP4921 pin 2 (CS) | Green | VSPI |
| GPIO18 | VSPI_SCK | MCP4921 pin 3 (SCK) | Yellow | VSPI |
| GPIO23 | VSPI_MOSI | MCP4921 pin 4 (SDI) | Blue | VSPI |
| GPIO25 | PWM_DRIFT | R2 left end (10kΩ RC filter) | White | PWM |
| GPIO12 | HSPI_MISO | MCP3202 pin 7 (Dout) | Purple | HSPI |
| GPIO13 | HSPI_MOSI | MCP3202 pin 6 (Din) | Blue | HSPI |
| GPIO14 | HSPI_SCK | MCP3202 pin 5 (CLK) | Yellow | HSPI |
| GPIO15 | HSPI_CS_ADC | MCP3202 pin 1 (CS) | Green | HSPI |
| 3V3 | DIGITAL_3V3 | DIGITAL_3V3 row | Red | Power |
| GND | Common GND | GND rail | Black | Ground |

▶  Use 10cm M-F jumper wires for ESP32-to-breadboard connections. For the RF path (MCP4921 VOUT → R1 → C6 → RESONATOR_IN), use solid-core wire cut to under 3cm to minimise parasitic inductance.

## E.8 — System Integration Test Sequence

| Step | Action | Expected Result | If Fails |
| --- | --- | --- | --- |
| 1 | Power on. Check both rails with multimeter. | ANALOG_3V3: 3.28-3.32V. DIGITAL_3V3: 3.28-3.32V. | Check AMS1117 wiring. Verify USB input > 4.75V. |
| 2 | Flash combined firmware. Open Serial Monitor at 921600 baud. | ARCS_INIT_OK prints within 3 seconds. No SPI errors. | Check all SPI pin definitions. Verify CS pins not swapped. |
| 3 | Observe Serial Monitor for 30 seconds without driving resonator. | MEASURE values printing at 20Hz. Values near 0.0 (no RF input). | If garbage: check baud rate. If frozen: check HSPI_CS_PIN=15. |
| 4 | Drive resonator at real f0 (from LTspice result: 555.6 kHz). | MEASURE values rise to 0.9-1.0 range within 100ms. | If <0.5: check ENVELOPE_OUT wire to MCP3202 pin 2. Check OPA2134 unity-gain feedback wire. |
| 5 | Change drive frequency to f0 + 50 kHz (605 kHz). | MEASURE values drop to 0.4-0.6 range. | If stays high: LC resonator not tuned. Verify L1 and C8 values. |
| 6 | Change GPIO25 PWM duty from 10% to 90% slowly. | MEASURE peak frequency shifts (varactor tuning). f0 moves ±5-15 kHz. | If no shift: check varactor polarity. Check R3 and C10 connection. |
| 7 | Run 500 consecutive MEASURE reads. Calculate std deviation. | Std deviation < 0.01 (i.e., < 1% amplitude noise). | If > 0.01: increase ADC_SAMPLES to 128. Add extra 10µF at MCP3202 VDD. |

## E.9 — Ground Rules for the Physical Build

- Single-point ground: Connect the analog GND and digital GND at ONE point only — the negative terminal of the MB102 PSU. Do not connect them at multiple points along the rails, as this creates ground loops that add noise to ADC readings.

- RF path wire length: All wires in the path from MCP4921 VOUT → R1 → C6 → L1/C8 tank → R4 → D2 → C_HOLD must be under 3cm. Use solid core wire cut to length, not flexible jumper wires.

- Decoupling placement: Place all 100nF decoupling capacitors as close as physically possible to each IC's VDD pin — within 3mm if the breadboard allows. Decoupling at 5cm distance is ineffective at 556 kHz.

- Varactor orientation: BB909 cathode (marked band) faces the LC tank (RESONATOR_IN). Anode faces R3. Reverse polarity means zero varactor tuning effect.

- OPA2134 feedback wire: The short wire from OPA2134 pin 6 (OUT) to pin 2 (IN−) must be present and confirmed with a continuity test before powering up Stage 4.

- Electrolytic polarity: Both 10µF electrolytic caps (C1, C3) have a positive lead. The positive lead connects to the 3.3V rail. Reversing polarity permanently damages the capacitor.

- SMD components: The SRR1280 inductor and all 0805 capacitors (8.2nF, 100pF, 1µF) must have berg strip pin leads soldered before insertion. Test mechanical stability by gently pressing — they must not rock.

ARCS Complete Simulation & Build Guide  |  V4 AUTHORITATIVE  |  DSCE 2025-26