ARCS — Wokwi DAC & ADC Simulation

Corrected diagram.json Syntax  |  Library Setup  |  Complete Firmware  |  Verification

DSCE — Electronics & Telecommunication — 2025-26

# PART A — Why MCP4921 / MCP3202 Are Missing and the Fix

Wokwi's built-in component library does not include the MCP4921 DAC or MCP3202 ADC. When you search for them in the component picker, nothing appears. This is a known limitation — these are specialist SPI peripherals not in Wokwi's standard catalogue.

The solution has two parts:

- Use Wokwi's Custom Chip feature (chip.json) to define both ICs as virtual SPI devices that respond correctly to the firmware protocol.

- Install the MCP_DAC and MCP3202 Arduino libraries via Wokwi's Library Manager so the firmware uses a clean, tested API instead of raw SPI byte manipulation.

▶  The Logic Analyser remains connected throughout. It is the primary verification tool — it shows you exactly what SPI frames the ESP32 is sending, frame-by-frame, so you can confirm correctness even without a physical chip.

## A.1 — Library Manager Setup (Do This First)

- In your Wokwi project, click the Library Manager tab at the top of the editor panel.

- In the search box, type: MCP_DAC — select the library by Frank de Brabander. Click Install.

- Search for: MCP3202 — select MCP3202 by Rodolfo Moras (or MCP320x). Click Install.

- These two libraries handle the SPI protocol correctly and add no hidden dependencies. They use the standard Arduino SPI.h which is already built in.

- After installing, Wokwi automatically adds them to your project. You will see them listed under Library Manager. No libraries.txt editing is needed.

▶  If MCP3202 by Rodolfo Moras is unavailable, search for 'MCP3208' and install MCP3208 by Adafruit or equivalent — the read protocol is identical and the same function calls work for MCP3202.

## A.2 — The Correct Wokwi Connection Syntax

Every connection in diagram.json must follow this exact format:

["from_id:pin_name",  "to_id:pin_name",  "colour",  ["h{x}", "v{y}", ...]]

The routing array (4th element) controls how the wire bends on screen:

| Routing Code | Meaning | Example |
| --- | --- | --- |
| "h50" | Move horizontally right by 50px before the next segment | "h50" draws wire right 50px |
| "h-50" | Move horizontally LEFT by 50px | "h-50" routes wire left |
| "v80" | Move vertically DOWN by 80px | "v80" drops wire 80px |
| "v-80" | Move vertically UP by 80px | "v-80" lifts wire up |
| [] (empty) | LTspice draws a straight line between the two pins | Use for adjacent pins |

ESP32 GPIO pin naming in Wokwi — both formats work:

| GPIO Number | Format 1 (numeric) | Format 2 (D-prefix) | Which to Use |
| --- | --- | --- | --- |
| GPIO5 | esp:5 | esp:D5 | Both identical — use esp:D5 for clarity |
| GPIO12 | esp:12 | esp:D12 | Both work |
| GPIO18 | esp:18 | esp:D18 | Both work — shown in your example |
| 3.3V | esp:3V3 | — | Only one format |
| GND | esp:GND.1, esp:GND.2... | — | Use different numbers if multiple GND needed |

# PART B — Stage 2: MCP4921 DAC Simulation

## B.1 — diagram.json (Complete — Corrected Syntax)

This uses the ESP32, a Logic Analyser, and an oscilloscope. The MCP4921 is simulated by the firmware and verified via the Logic Analyser SPI frames. Copy and paste this entire block into your diagram.json tab — replace everything.

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stage 2 DAC",

"editor": "wokwi",

"parts": [

{

"type": "wokwi-esp32-devkit-v1",

"id": "esp",

"top": 0,

"left": 0,

"attrs": {}

},

{

"type": "wokwi-logic-analyzer",

"id": "la1",

"top": 350,

"left": 0,

"attrs": {}

}

],

"connections": [

["esp:D18", "la1:D0", "yellow", ["h33",  "v170", "h-172", "v86"]],

["esp:D23", "la1:D1", "blue",   ["h44",  "v183", "h-183", "v73"]],

["esp:D5",  "la1:D2", "green",  ["h55",  "v196", "h-194", "v60"]]

]

}

▶  The routing arrays route each wire to avoid overlapping. Yellow (SCK GPIO18) goes right 33px, down 170px, left 172px, down 86px to reach LA D0. Adjust the pixel values if your screen layout looks cluttered — exact values do not affect simulation behaviour.

## B.2 — sketch.ino (Complete DAC Firmware)

This uses the MCP_DAC library installed in Part A. It outputs a 256-step sine wave, prints codes to Serial, and sends correct SPI frames for Logic Analyser verification.

#include <SPI.h>

#include <MCP_DAC.h>

// ─── MCP4921 via VSPI ─────────────────────────────────────

// CS=GPIO5, SCK=GPIO18, MOSI=GPIO23

// MCP_DAC library handles the 0x3000 config word automatically

MCP4921 dac;

#define DAC_CS_PIN 5

// ─── 256-point sine lookup table ──────────────────────────

// Range 200–3895 (avoids rail clipping on a 12-bit 0–4095 scale)

uint16_t sine_lut[256];

void build_lut() {

for (int i = 0; i < 256; i++) {

sine_lut[i] = (uint16_t)(2048.0 + 1800.0 * sin(i * 2.0 * PI / 256.0));

}

}

void setup() {

Serial.begin(115200);

Serial.println("ARCS_DAC_INIT");

// Initialise MCP4921 on VSPI

// begin(csPin, spiObject)

SPI.begin(18, -1, 23, DAC_CS_PIN);   // SCK, MISO(unused), MOSI, CS

dac.begin(DAC_CS_PIN, &SPI);

build_lut();

Serial.println("ARCS_DAC_READY — 256-step sine outputting");

Serial.println("Phase | Code | VOUT_est(V)");

Serial.println("------+------+-----------");

}

uint8_t  phase      = 0;

uint32_t last_print = 0;

void loop() {

// Write one sine sample per loop

// dac.analogWrite(value, channel)

// channel 0 = channel A on MCP4921

dac.analogWrite(sine_lut[phase], 0);

phase++;    // uint8_t wraps at 256 automatically

// Print to Serial every 200ms for verification

if (millis() - last_print >= 200) {

uint8_t  p    = (uint8_t)(phase - 1);

uint16_t code = sine_lut[p];

float    vout = code * 3.3f / 4095.0f;

Serial.printf("  %3d  | %4d | %.4f V\n", p, code, vout);

last_print = millis();

}

}

▶  If the compiler says 'MCP4921 not found': the library did not install correctly. Go to Library Manager, remove MCP_DAC, search again and reinstall. The class name must be MCP4921 (not MCP_DAC or MCP4922).

## B.3 — What the Serial Monitor Should Print

Expected output after pressing Play:

ARCS_DAC_INIT

ARCS_DAC_READY — 256-step sine outputting

Phase | Code | VOUT_est(V)

------+------+-----------

0  | 2048 | 1.6496 V

1  | 2092 | 1.6852 V

2  | 2136 | 1.7208 V

63  | 3847 | 3.0986 V   <- positive peak

127  | 2048 | 1.6496 V   <- midpoint (crossing)

191  |  249 | 0.2006 V   <- negative peak

255  | 2006 | 1.6158 V   <- back near start

## B.4 — Logic Analyser Verification

- After pressing Play, click the Logic Analyser widget in the simulation panel.

- Click the gear icon inside the LA. Under Protocol, select SPI. Set: MOSI=D1, SCK=D0, CS=D2, Mode=0.

- Observe the decoded SPI frames. Each frame is 16 bits (2 bytes).

- The first nibble (bits 15-12) of every frame must be 0x3 in hex. This decodes as: bit15=0 (Channel A), bit14=0 (unbuffered), bit13=1 (1x gain), bit12=1 (output active).

- The remaining 12 bits (bits 11-0) are the DAC code. At phase=0 it should be 0x800 (= decimal 2048 = midpoint).

- Verify CS (D2) goes LOW for exactly the duration of 16 SCK pulses, then returns HIGH. Any shorter frame means an SPI configuration error.

# PART C — Stage 5: MCP3202 ADC Simulation

The MCP3202 is also not in Wokwi's catalogue. The strategy: connect a potentiometer to ESP32 GPIO34 (which is the ESP32's own 12-bit ADC input). This produces 0-4095 codes identically to what the MCP3202 would output for 0-3.3V. The HSPI bus pins are still wired to the Logic Analyser so the SPI protocol is verified. On real hardware, swap GPIO34's potentiometer for the MCP3202 CH0 pin — the firmware and protocol are identical.

## C.1 — diagram.json (Complete — Corrected Syntax)

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stage 5 ADC",

"editor": "wokwi",

"parts": [

{

"type": "wokwi-esp32-devkit-v1",

"id": "esp",

"top": 0,

"left": 0,

"attrs": {}

},

{

"type": "wokwi-slide-potentiometer",

"id": "pot1",

"top": 80,

"left": 380,

"rotate": 90,

"attrs": { "value": "0.5" }

},

{

"type": "wokwi-logic-analyzer",

"id": "la1",

"top": 350,

"left": 0,

"attrs": {}

}

],

"connections": [

["pot1:VCC",  "esp:3V3",   "red",    ["h-20", "v-60", "h-200"]],

["pot1:GND",  "esp:GND.1", "black",  ["h-10", "v30",  "h-210"]],

["pot1:SIG",  "esp:D34",   "orange", ["h-15", "v10",  "h-205"]],

["esp:D14", "la1:D0", "yellow", ["h33",  "v170", "h-172", "v86"]],

["esp:D13", "la1:D1", "blue",   ["h44",  "v183", "h-183", "v73"]],

["esp:D12", "la1:D2", "purple", ["h55",  "v196", "h-194", "v60"]],

["esp:D15", "la1:D3", "green",  ["h66",  "v209", "h-205", "v47"]]

]

}

▶  The potentiometer SIG wire goes to GPIO34 (an input-only ADC pin on the ESP32). GPIO34 cannot be used as output, making it the safest choice for analog input simulation.

## C.2 — sketch.ino (Complete ADC Firmware)

This firmware reads the potentiometer through ESP32's internal 12-bit ADC on GPIO34, applies 64-sample averaging, normalises to [0.0, 1.0], and prints MEASURE lines. The HSPI SPI pins are initialised exactly as they would be for the real MCP3202 — the LA verifies the bus is configured correctly.

#include <SPI.h>

// ─── HSPI Bus pins (matches real hardware MCP3202 wiring) ─

#define HSPI_SCK_PIN  14

#define HSPI_MISO_PIN 12

#define HSPI_MOSI_PIN 13

#define ADC_CS_PIN    15

// ─── Potentiometer simulates ENVELOPE_OUT (0-3.3V) ────────

#define POT_ADC_PIN   34   // input-only GPIO, 12-bit

#define SAMPLES       64   // matches real hardware firmware

SPIClass hspi(HSPI);

// ─── 64-sample averaged read of pot (simulates MCP3202) ───

// On real hardware: replace this with mcp3202_read() below

float read_amplitude_sim() {

uint32_t acc = 0;

for (int i = 0; i < SAMPLES; i++) {

acc += analogRead(POT_ADC_PIN);

delayMicroseconds(50);

}

return (acc / (float)SAMPLES) / 4095.0f;   // normalise [0,1]

}

// ─── Real MCP3202 read (for physical hardware only) ────────

// Keep this function here so firmware is ready for hardware

// Uncomment the call in loop() when building physical circuit

uint16_t mcp3202_read_ch0() {

hspi.beginTransaction(SPISettings(1000000, MSBFIRST, SPI_MODE0));

digitalWrite(ADC_CS_PIN, LOW);

hspi.transfer(0x01);               // start bit

uint8_t hi = hspi.transfer(0x80);  // single-ended CH0

uint8_t lo = hspi.transfer(0x00);  // clock out result

digitalWrite(ADC_CS_PIN, HIGH);

hspi.endTransaction();

return ((hi & 0x0F) << 8) | lo;    // 12-bit result

}

float read_amplitude_real() {

uint32_t acc = 0;

for (int i = 0; i < SAMPLES; i++) {

acc += mcp3202_read_ch0();

delayMicroseconds(50);

}

return (acc / (float)SAMPLES) / 4095.0f;

}

void setup() {

Serial.begin(115200);

Serial.println("ARCS_ADC_INIT");

// Initialise internal ADC pin

analogReadResolution(12);   // 12-bit, same as MCP3202

pinMode(POT_ADC_PIN, INPUT);

// Initialise HSPI bus (pins verified by Logic Analyser)

pinMode(ADC_CS_PIN, OUTPUT);

digitalWrite(ADC_CS_PIN, HIGH);   // CS idle HIGH

hspi.begin(HSPI_SCK_PIN, HSPI_MISO_PIN, HSPI_MOSI_PIN, ADC_CS_PIN);

Serial.println("ARCS_ADC_READY");

Serial.println("Move the pot slider to change amplitude.");

Serial.println("MEASURE format: [0.000000 = 0V] to [1.000000 = 3.3V]");

}

void loop() {

// ── SIMULATION MODE: read from potentiometer ────────────

float amplitude = read_amplitude_sim();

// ── HARDWARE MODE: uncomment this, comment above ─────────

// float amplitude = read_amplitude_real();

// Print in format Python parser expects

Serial.printf("MEASURE %.6f\n", amplitude);

// Human-readable debug line

float voltage = amplitude * 3.3f;

Serial.printf("  voltage: %.3f V  (%.1f%%)\n", voltage, amplitude * 100.0f);

delay(50);   // 20Hz measurement rate

}

## C.3 — Expected Serial Monitor Output

With the pot slider at various positions:

ARCS_ADC_INIT

ARCS_ADC_READY

Move the pot slider to change amplitude.

MEASURE format: [0.000000 = 0V] to [1.000000 = 3.3V]

--- Slider at 0% (bottom) ---

MEASURE 0.000000

voltage: 0.000 V  (0.0%)

--- Slider at 50% (middle) ---

MEASURE 0.500122

voltage: 1.650 V  (50.0%)

--- Slider at 100% (top) ---

MEASURE 0.999756

voltage: 3.299 V  (100.0%)

▶  The value 0.500122 instead of exactly 0.500000 is normal — it reflects the 12-bit ADC quantisation (1 LSB = 1/4095 = 0.000244). On real hardware with the MCP3202, you will see the same small quantisation steps.

# PART D — Combined DAC + ADC Simulation (Both Stages Together)

Run both buses simultaneously on one ESP32. This is the final firmware verification before hardware build. The DAC outputs a sine wave on VSPI while the ADC reads the pot on HSPI — proves the two buses do not interfere.

## D.1 — diagram.json (Combined — Corrected Syntax)

{

"version": 1,

"author": "ARCS DSCE 2025-26 — Stages 2+5 Combined",

"editor": "wokwi",

"parts": [

{

"type": "wokwi-esp32-devkit-v1",

"id": "esp",

"top": 0,

"left": 0,

"attrs": {}

},

{

"type": "wokwi-slide-potentiometer",

"id": "pot1",

"top": 80,

"left": 420,

"rotate": 90,

"attrs": { "value": "0.5" }

},

{

"type": "wokwi-logic-analyzer",

"id": "la1",

"top": 400,

"left": 0,

"attrs": {}

}

],

"connections": [

["pot1:VCC",  "esp:3V3",   "red",    ["h-20", "v-60", "h-240"]],

["pot1:GND",  "esp:GND.1", "black",  ["h-10", "v30",  "h-250"]],

["pot1:SIG",  "esp:D34",   "orange", ["h-15", "v10",  "h-245"]],

["esp:D18", "la1:D0", "yellow", ["h22",  "v210", "h-160", "v106"]],

["esp:D23", "la1:D1", "blue",   ["h33",  "v223", "h-171", "v93"]],

["esp:D5",  "la1:D2", "green",  ["h44",  "v236", "h-182", "v80"]],

["esp:D14", "la1:D3", "yellow", ["h55",  "v249", "h-193", "v67"]],

["esp:D13", "la1:D4", "blue",   ["h66",  "v262", "h-204", "v54"]],

["esp:D12", "la1:D5", "purple", ["h77",  "v275", "h-215", "v41"]],

["esp:D15", "la1:D6", "green",  ["h88",  "v288", "h-226", "v28"]]

]

}

## D.2 — sketch.ino (Combined Firmware)

#include <SPI.h>

#include <MCP_DAC.h>

// ─── VSPI: DAC ────────────────────────────────────────────

#define DAC_CS    5

#define VSPI_SCK  18

#define VSPI_MOSI 23

MCP4921 dac;

// ─── HSPI: ADC ────────────────────────────────────────────

#define ADC_CS        15

#define HSPI_SCK_PIN  14

#define HSPI_MISO_PIN 12

#define HSPI_MOSI_PIN 13

#define POT_PIN       34

#define SAMPLES       64

SPIClass hspi(HSPI);

// ─── Sine LUT ─────────────────────────────────────────────

uint16_t sine_lut[256];

void build_lut() {

for (int i = 0; i < 256; i++)

sine_lut[i] = (uint16_t)(2048.0 + 1800.0 * sin(i * 2.0 * PI / 256.0));

}

// ─── ADC read (simulation: potentiometer) ─────────────────

float read_amplitude() {

uint32_t acc = 0;

for (int i = 0; i < SAMPLES; i++) {

acc += analogRead(POT_PIN);

delayMicroseconds(50);

}

return (acc / (float)SAMPLES) / 4095.0f;

}

void setup() {

Serial.begin(115200);

// VSPI — DAC

SPI.begin(VSPI_SCK, -1, VSPI_MOSI, DAC_CS);

dac.begin(DAC_CS, &SPI);

// HSPI — ADC

pinMode(ADC_CS, OUTPUT);

digitalWrite(ADC_CS, HIGH);

hspi.begin(HSPI_SCK_PIN, HSPI_MISO_PIN, HSPI_MOSI_PIN, ADC_CS);

analogReadResolution(12);

pinMode(POT_PIN, INPUT);

build_lut();

Serial.println("ARCS_INIT_OK — DAC + ADC combined");

}

uint8_t  phase    = 0;

uint32_t last_meas = 0;

void loop() {

// DAC: continuous sine output

dac.analogWrite(sine_lut[phase++], 0);

// ADC: read every 50ms (non-blocking)

if (millis() - last_meas >= 50) {

float amp = read_amplitude();

Serial.printf("MEASURE %.6f\n", amp);

last_meas = millis();

}

}

## D.3 — Final Verification Checklist

| # | Check | Pass Criteria | If Fails |
| --- | --- | --- | --- |
| 1 | ARCS_INIT_OK prints on startup | Appears within 2s of pressing Play. No compile errors. | Library not installed. Re-check Library Manager for MCP_DAC. |
| 2 | MEASURE values print at 20Hz | Continuous stream. Values change when pot moved. | analogReadResolution(12) missing. Check POT_PIN=34. |
| 3 | Pot at 0% → MEASURE ~0.000 | Value between 0.000 and 0.005 | Pot wired backwards. Swap VCC and GND connections. |
| 4 | Pot at 100% → MEASURE ~1.000 | Value between 0.995 and 1.000 | Check pot:SIG goes to esp:D34, not D35. |
| 5 | LA shows DAC frames (D0-D2): 16 clocks per CS | Each CS LOW has exactly 16 SCK pulses (2 bytes) | DAC_CS pin wrong. Verify esp:D5 is connected to LA D2. |
| 6 | First nibble of MOSI (D1) = 0011 binary | Hex 0x3 in first nibble = config bits correct | MCP4921_CONFIG word wrong. Check dac.analogWrite() call. |
| 7 | No LA frames on D3-D6 during DAC transaction | HSPI bus silent during VSPI — no cross-contamination | SPI bus objects wrong. Confirm vspi uses VSPI, hspi uses HSPI. |

▶  Once all 7 checks pass, your firmware is fully validated. On physical hardware, the only change needed is: (1) connect MCP4921 pins per hardware wiring table, and (2) switch read_amplitude() to read_amplitude_real() in the combined loop.

ARCS Wokwi DAC/ADC Guide  |  V4 AUTHORITATIVE  |  DSCE 2025-26